## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)/(SH404) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of steam tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the first law of thermodynamics and discuss its importance. |  |
| b) | Define specific heat, and explain specific heat at constant volume. |  |
| c) | Draw the dual air standard cycle on a PV diagram and explain the processes. |  |
| d) | Give the classification of IC engines. |  |
| e) | List a few applications of steam on ships. |  |
| f) | List the four important components of the refrigeration cycle and their functions. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain thermal equilibrium and zeroth law of thermodynamics. |  |
| b) | Sate and explain Charles law. A quantity of gas at 0C occupies 0.6m3. What would be its volumes at 400C, if the pressure is same at both temperatures? |  |
| c) | A certain quantity of air in a closed vessel of fixed volume of 0.15m3 exerts a pressure of 1MPa at 500K. If the vessel is cooled so that the pressure falls to 350KPa, determine: i) the final temperature ii) the change in internal energy iii) the heat transferred. Take Kv = 0.7165 KJ/KgK, R = 0.287KJ/KgK. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State what do you understand by i) an isothermal process ii) an adiabatic process. Represent the processes on P-V and T-S diagram. |  |
| b) | One Kg of air at 150 KPa and 30C undergoes a constant pressure process until the volume is trippled. Determine i) the change in internal energy and ii) change in enthalpy. Take Kp = 1.0035KJ/KgK and Kv = 0.7165KJ/KgK for air. |  |
| c) | A four cylinder engine has 68 mm bore and 75mm stroke. The compression ratio is 8. Determine the cubic capacity of the engine and the clearance volume of each cylinder. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by volumetric efficiency of an IC engine? Distinguish between two stroke and four stroke IC engine. |  |
|  |  | …….2/- |
|  | -2- |  |
| b) | In a test on a single cylinder oil engine working on four stroke cycle, the following readings were taken: Indicated mean effective pressure : 540 Kpa Piston stroke length : 30 cm Engine speed : 4 rps Mechanical Efficiency : 73% Mass of fuel supplied : 8 Kg/hr Calorific value of oil : 42,000KJ/Kg Determine i) Indicated Power ii) Brake Power iii) Brake Thermal efficiency. |  |
| c) | With simple sketches explain the parallel and counter flow heat exchangers. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Use steam tables and find the following: - Dryness fraction of steam at a pressure of 5bar having enthalpy of 2,500 KJ/Kg. - The condition of steam at a pressure of 8 bar and volume 0.22m3/Kg. - The volume of one Kg of steam at a pressure of 9 bar when the steam is 80% dry. |  |
| b) | Explain the term dryness fraction of steam. How much heat is needed to convert 5 Kg of water at 45C into steam at pressure of 4bar and 80% dry. |  |
| c) | Represent the following process on the P-H diagram and explain their importance: i) Wet compression ii) Dry compression iii) Refrigerating effect. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on refrigerants and the harmful effects of refrigerants on environment. |  |
| b) | An ammonia refrigerator works between –10C and 38C, the vapour leaving the compressor 100% dry. There is no undercooling. Sketch the cycle on the T-S diagram. The refrigerant leaves the compressor with a discharge temperature of 380.7K. Using the table given below determine i) COP ii) mass of refrigerant circulated per TR. Temp C; Enthalpy KJ/Kg. Liquid (hf); Latent (hfg); Vapour (hg). -10 38; -808.71 -581.57; 1296.47 1108.34; 487.76 526.77. Take Kp for liquid = 4.75KJ/KgK Kp for Vapour = 3.00 KJ/KgK. |  |
| c) | Write short notes on: i) Two stroke cycle engines ii) Fourier law of heat transfer |  |

***********