## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./FT&EE

Subject: Elements of Civil Engineering (CS311) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is ranging? Stating different types of ranging, mention when each types is used. |  |
| b) | Explain how do you detect the presence of local attraction at a survey station. |  |
| c) | What is bench mark? State different types of B.M. used in surveying. |  |
| d) | Distinguish between load bearing and framed structure. |  |
| e) | What is bearing capacity of soil? State how it affects the selection of different types of foundation. |  |
| f) | Stating its significance, list out various fire protection measures followed in a multistoried building. |  |
| g) | Sketch conventional signs used to represent various units of building in a drawing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the aid of neat sketch, explain the procedure adopted in reciprocal ranging. |  |
| b) | The following are the Fore bearings observed in a closed traverse. Calculate the interior angles of traverse. Line Fore Bearing AB N 45 30 E BC S 60 0 E CD S10 30 W DA N75 45 W |  |
| c) | With the aid of neat labelled sketch, explain various components of dumpy level. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following consecutive readings were taken with a levelling instrument at 20m intervals. 2.125, 1.710, 0.720, 3.350, 2.675, 2.170, 1.665, 0.965, 0.555, 1.685, 2.355 and 3.730. The instrument was shifted after the fourth and eight readings. Enter the readings in a level page and calculate the RL of all points, if RL of first point is 101.300m. |  |
| b) | With the aid of neat sketch, explain the procedure involved in prolonging a straight line using a theodolite. |  |
| c) | Describe the procedure involved to verify the verticality of a tower using theodolite. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe in detail the procedure involved in construction of mosaic flooring. |  |
| b) | With the aid of neat sketch explain the following: i) Spread footing ii) Isolated footing. |  |
| c) | Stating their suitability, draw neat sketches of various types of roof trusses used. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the aid of neat sketch explain the various components of a sloping roof: i) Ridge ii) Eaves iii) Gable iv) Hip. |  |
| b) | Describe the various methods of curing adopted for concrete structures. |  |
| c) | Distinguish between the following: - R.CC and pre-stressed concrete - Screeding and needle vibration. |  |
| Q.No.6. | The following are the construction details of a single room with attached toilet. Size of room 3.2m x 4.0m Attached toilet 1.25m x 1.5m Height of roof above floor –3.0m Height of Plinth above G.L- 0.5m Doors 1.0m x 2.0m Windows 1.25m x 1.5m Ventilator 1.0m x 0.5m Parapet wall 0.2m thick and 0.9m height above roof wall thickness 0.23m Roof-R.CC slab 120mm thick Chajja projection 0.6m from face of wall Roof projection alround 0.3m Draw the plan and section passing through the window opening. Use 1:50 scale. | (12) |

***********