## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: A.C. Machines(4113)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give two points of distinction between the construction of rotor in case of Squirrel Cage induction motor and slip ring Induction motor. |  |
| b) | How do you decide about the direction of rotation of 3 phase Induction motor? State the expression for frequency of rotor induced e.m.f. in case of 3 phase Induction motor. Also state its value when the rotor is stationary. |  |
| c) | State the condition for occurrence of maximum torque in case of 3 phase Induction motor during starting & running condition. |  |
| d) | State the relationship between starting torque and maximum torque in case of 3 phase Induction motor indicating clearly each term used in the expression |  |
| e) | Classify the rotors in the construction of an alternator and give two points of distinction between them. |  |
| f) | State the effect of change in input power on the synchronisation on an alternator. |  |
| g) | List out the specifications for procurement of an alternator. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw the phasor diagram of 3 phase Induction motor at rated load induction, indicating clearly each phasor used. |  |
| b) | State the expression for torque developed by rotor in 3 phase Induction motor indicating clearly each term used in the expression. |  |
| c) | Draw the power flow diagram of 3 phase Induction motor, indicating clearly losses occurring at each stage of power flow. Also state the relationship between: i) Rotor copper loss & rotor input ii) Gross Mechanical power developed in rotor and rotor input. |  |
| d) | A 3 phase, 6 pole, 50 Hz operated Induction motor develops 3700 watts at 950 r.p.m. If total stator losses are 300 watts & friction and windage losses are 200 watts, determine the motor efficiency. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is the necessity of starter in 3 phase Induction motor? State the type of starter used in slip ring Induction motor and draw its diagram. |  |
| b) | IS 1 Induction motor self starting ? Give your choice & justify it. |  |
| c) | Classify single phase Induction motors. Explain any one of such motor in brief with the help of neat diagram. |  |
| d) | Draw power circuit diagram as well as control circuit diagram for D.O.L. starter. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Define hunting phenomenon. Where does it occur? How can it be minimized? |  |
| b) | State the expression for e.m.f. induced across the armature of alternator, indicating clearly each term used in the expression. How can the magnitude of e.m.f. induced be increased? |  |
| c) | State the methods used for synchronising of alternators. Also explain any one of such method in brief with the help of neat diagram. |  |
| d) | Two similar 400V, 3 phase alternators share equal KW power delivered to a balanced 3, 50 KW at 0.8 p.f. lag load. If the power factor of one machine is 0.95 lag. Find the power factor and current supplied by the other machine. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Define the following terms: i) Synchronous reactance ii) Armature resistance iii) Synchronous Impedance in alternator. State the methods used to determine the above parameters. What is the relationship between d.c. and a.c. armature resistance? |  |
| b) | Draw the phasor diagram of alternator on load condition, operating at lagging, leading and unity power factor. Also state the methods used to determine voltage regulation in an alternator. |  |
| c) | A 3 phase star connected 1000KVA, 11000V alternator has rated current of 52.5A. The a.c. resistance of winding per phase is 0.45Ω. The O.C. and S.C. tests performed gave following results: O.C. Tests – Field current = 12.5A, Line voltage = 422V S.C. Test - Field current = 12.5A, Line current = 52.5A Determine full load voltage regulation of alternator at 0.8 p.f. lagging. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain in brief, with the help of neat diagram, principle of operation of synchronous motor. |  |
| b) | Explain in brief the concept of synchronous condenser. |  |
| c) | Draw a circuit diagram of an universal motor. Can such motor run safely on d.c. supply? Give 4 of its applications. |  |
| d) | Classify single phase synchronous motor and explain any one of such motor with the help of neat diagram. |  |

***********