## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: Substation Practice(5105)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare outdoor substation with indoor substation. |  |
| b) | State the merits and demerits of single busbar system. |  |
| c) | Define ratio error and phase angle error for a C.T. |  |
| d) | What is an earthing switch? Explain its function in a substation. |  |
| e) | Explain the different types of control and relay panels in brief. |  |
| f) | List and explain the function of any three components of a HVDC substation. |  |
| g) | - List any three types of traction substations. - What is a mimic diagram? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain classification of substations based on the service requirement. |  |
| b) | With a neat diagram explain double bus bar arrangement. Also state the function of bus coupler in the arrangement. |  |
| c) | Explain why secondary circuit of C.T. should never be opened when its primary is energized. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat single line diagram of 33/11KV substation and label its components. |  |
| b) | Explain in brief block rate tariff and power factor tariff. |  |
| c) | A 400V, 50Hz, 3 line delivers 200 KW at 0.8 p.f. lagging. It is desired to raise the line power factor to unity by installing shunt capacitors. Calculate capacitance of each unit if they are connected i) in star ii) in delta. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare isolated neutral system with grounded neutral system. |  |
| b) | With a neat diagram explain the working of explusion type arrestors. Give any two advantages of the same. |  |
| c) | Explain in brief how overhead ground wires provide protection to lines against lightening strokes. List any two drawbacks of this type of protection system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the general construction of a three core cable. |  |
| b) | With a neat diagram explain the construction of Gas pressure cables. Give any two merits of the same. |  |
| c) | Explain in brief any two methods of laying underground cables. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is silica gel? Where is it used? Explain how it is reactivated? |  |
| b) | Explain any three maintenance tests carried out on insulation oil of a transformer. |  |
| c) | Explain in brief different methods of charging lead acid batteries. |  |

***********