## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam Tables, and Mollier’s charts is permissible.

| Q.No.1. | Answer any five of the following. | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe i) geothermal energy ii) Solar energy. |  |
| b) | What do you understand by ‘minimum air’ and ‘excess air’ in context of combustion? |  |
| c) | State second law of thermodynamics according to Kelvin-Planck. What do you understand by the term degradation of energy? |  |
| d) | Differentiate between specific heat at constant pressure and specific heat at constant volume. |  |
| e) | Define the terms i) Sensible heat of water ii) Dryness fraction of steam. |  |
| f) | State three merits and three limitations of wind energy. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the Boy’s gas calorimeter and the procedure to calculate Higher calorific value. |  |
| b) | Two Kgs of ideal gas is heated from 28C to 98C. Assuming R=0.264 KJ/Kg K and  = 1.2 for the gas. Find the following: i) Specific heat ii) Change in enthalpy iii) Work done. |  |
| c) | Define and represent the following processes on P-V and T-S diagram: i) Isobaric process ii) Isochoric process. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With a neat sketch explain the use of any steam calorimeter for measuring dryness fraction of steam. |  |
| b) | Find the Entropy of 1 Kg of steam at 15 bar in the following cases: i) When the steam is wet having dryness fraction of 0.6. ii) When the steam is superheated to 300C. Take specific heat for superheated steam as 2.3 KJ/KgK. |  |
| c) | In an experiment, it was found that the steam enters a throttling calorimeter at a pressure of 12 bar. After throttling, the pressure and temperature was measured as 1.013 bar and 115C respectively. Estimate the dryness fraction of steam. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is the purpose of Fusible plug? Explain its construction with the help of neat sketch. |  |
| b) | Describe with a neat sketch the construction and working of a Lancashire boiler. |  |
| c) | Explain with a neat sketch the construction and working of a Benson boiler. |  |
|  | -2- | ……2/- |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is the function of an economizer? Describe the construction of an economizer with the help of a neat sketch. |  |
| b) | Show modified Rankine cycle on P-V and T-S diagrams and explain the processes involved. State the reasons for modifying the Rankine cycle for operation of steam engines. |  |
| c) | With the help of a neat sketch explain the construction and working of a Vane type compressor. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Applications of compressed air |  |
| b) | Multi-stage air compressors |  |
| c) | Centrifugal compressor |  |
| d) | Boiler draught |  |
| e) | Avogadro’s law |  |

***********