## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Common

Subject: Elements of Civil Engineering(3003) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain method of stepping on sloping ground with neat sketch. |  |
| b) | Define i) Bearing of a line ii) Local attraction iii) Back Bearing. |  |
| c) | Define i) Face left ii) Face Right iii) Ranging. |  |
| d) | If Fore bearing of a line is 14530, what is its Back bearing? |  |
| e) | What is safe bearing capacity of soil? What is its significance? |  |
| f) | What are the ingredients of concrete? List the stages involved in preparing concrete. |  |
| g) | What is Thermal Insulation? List down the materials used. |  |
| h) | Define i) Acoustics ii) Foundation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Indirect Ranging with neat sketch. |  |
| b) | Explain Collimation and rise and fall method for calculating RL of points. |  |
| c) | Write a short note on Profile levelling. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Describe the procedure of measuring vertical angle by using Theodolite. | (4) |
|  | ### OR |  |
| a) | Explain briefly different types of Trusses used for Roofing. | (4) |
| b) | The following readings are successively taken with a level. 0.355, 0.485, 0.625, 1.755, 1.895, 2.350, 1.780, 0.345, 0.685, 1.230 & 2.150. The Instrument was shifted after the 4th & 7th reading. Prepare a level book & calculate RL of different points. The RL of the 1st point is 245.50m. | (8) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) What is the purpose of providing foundation? ii) Enumerate the different types of Roof coverings. |  |
| b) | Write short note on types of flooring. |  |
| c) | Explain briefly different methods of transporting concrete. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain briefly sound insulation and materials used for sound insulation. |  |
| b) | Write short note on types of Foundations & draw neat sketches of any two types. |  |
| c) | Describe the procedure for measuring horizontal angle by Theodolite. |  |
| Q.No.6. | Using the line plan given in Fig.1, draw to a suitable scale i) Plan ii) Elevation iii) Section. Assume any suitable data as per PDA Rules. | 4 + 4 + 4 |