## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Control Engineering(4219)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief reset windup. |  |
| b) | Explain any one application of a dc tachometer. |  |
| c) | Draw block diagram of dc motor. State the transfer function. |  |
| d) | Draw the block diagram of a positional servomechanism system. |  |
| e) | What is meant by stability in a feedback system? |  |
| f) | Give advantages and disadvantages of using closed loop system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With help of block diagram explain the working of an automatic tank level control system. |  |
| b) | Draw schematic diagram of synchro control receiver and describe its working. |  |
| c) | Explain with a neat diagram separately excited DC generator. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Obtain the transfer function for the system represented in fig.1 and draw reduced block diagram. |  |
| b) | Find the Laplace inverse of i) v(s)=8s+4s2+6s+13 ii) F(s)=5s+12s2+8s+3 iii) v(s)=200s2+100 |  |
| c) | What is shaft encoder? Explain any one type of shaft encoder. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Obtain the transfer function of L-R-C system in fig.2. |  |
| b) | A step input position servomechanism system has the following constants R=1, K=2, J=1, F=2 and h=1. Find m(t). |  |
| c) | Given a second order servomechanism with K=9, J=1 & F=3, find the i) natural frequency ii) damping ratio iii) damped frequency & iv) time constant. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Neatly draw second order servo system and write its transfer function. |  |
| b) | Consider a sixth order system with the characteristics equation s6+2s5+8s4+12s3+20s2+16s+16=0. Determine the stability of the above system using Routh criterion. |  |
| c) | Given the transfer function G(s)=100ss+10, determine the gain from Bode plot. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the polar plot for G(s)=1s(1+sa). |  |
| b) | Determine the stability of the feedback system whose open loop transfer function is given below using Nyquist plot. G(s)H(s)=k(T1s+1)(T2+1) |  |
| c) | Compare open loop & closed loop control systems. |  |