## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg./A

Subject: Theory of Machines(4039)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by completely constrained motion, incompletely constrained motion and successfully constrained motion? Give example of each. |  |
| b) | What is Turning moment diagram? State the function of a flywheel. |  |
| c) | Define slip with reference to belt drive. Why slip occurs in belt drive? |  |
| d) | What is sensitiveness and Hunting in Governors? |  |
| e) | What is the function for a Gyroscope? State its different applications. |  |
| f) | What do you understand by i) Period ii) Cycle iii) Frequency in a vibration? |  |
| g) | With a neat sketch explain the working of Internal expanding brakes. |  |
| h) | State the law of Gearing. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | A Double slider-crank mechanism is shown in Fig.1. The crank OA rotates at constant angular velocity of 10 rad/s. The links OA, AB and AC are 100mm, 200mm and 200mm long respectively. Determine i) Velocity of each slider ii) Angular velocity of connecting rod. | (6) |
|  | ### OR |  |
| a) | The lengths of various links of a mechanism shown in Fig.2 are as follows: OA = 150mm, CD = 125mm, AC = 600mm, BD = 500mm, CQ = QD = 145mm, OQ = 625mm. If the crank OA rotates at 60 rpm in counterclockwise direction, determine the velocity of slider B and angular velocity of the link BD when the crank has turned an angle of 45 with the vertical. | (6) |
| b) | Explain kliens constructions method for finding velocity and acceleration of piston in reciprocating engine mechanism. | (6) …….2/- |
|  | -2- |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A load of 10KN is raised by means of a screw jack having a square threaded screw of 12mm pitch and a mean diameter of 50mm. If a force of 100N is applied at the end of a lever to raise a load, what should be the length of the lever used? Take coefficient of friction as 0.15. What is the mechanical advantage obtained? State whether the screw is self-locking. |  |
| b) | A single plate clutch is required to transmit 8KW at 1000 rpm. The axial pressure is limited to 70KN/m2. The mean radius of plate is 4.5 times the radial width of the friction surface. If both the sides of the plate are effective and coefficient of friction is 0.25, find the inner and outer radii of the plate and the mean radius. |  |
| c) | A conical pivot supports a load of 20 KN. The angle of cone is 120 and the intensity of pressure is not to exceed 3.5 bar. The external radius is 3 times the internal radius. Find the diameter of the bearing surface. If coefficient of friction is 0.065 and rpm of shaft is 120, what power in KW is absorbed by friction? |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Determine the maximum power transmitted by a V-Belt drive having the included V-groove angle of 35. The belt used is 18mm deep with 18mm maximum width and weight 300gms per metre length. The angle of Lap is 145 and the maximum permissible stress is 1.5 MPa. Take coefficient of friction to be 0.2. | (6) |
|  | ### OR |  |
| a) | Derive an expression for ratios of Tensions T1/T2 = e in case of flat belt drive. | (6) |
| b) | With the help of neat sketch explain the construction and working of Porter Governor | (6) |
|  | ### OR |  |
| b) | With the help of neat sketch explain the construction and working of Withworth quick return mechanism. | (6) |

………3/-

-3-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Draw the profile of a cam with roller reciprocating follower. The follower moves with uniformly accelerated and decelerated motion during descent. Least radius of cam is 50mm, angle of ascent is 48, angle of dwell between ascent and descent is 42, angle of descent is 60, the lift of the follower is 40mm, diameter of roller is 30mm. Follower motion during ascent is Simple Harmonic motion(SHM). Distance between line of action of follower and axis of cam is 20mm. If the cam rotates at 360 rpm anticlockwise, find the maximum velocity and acceleration of the follower during descent. | (9) |
| b) | What is meant by Train-value of a Gear Train? Obtain an expression for Train-value for a simple Gear Train. | (3) |
|  | ### OR |  |
| b) | In a compound Gear Train shown in Fig.3 the power is transmitted from a motor shaft to output shaft. The motor shaft is connected to gear 1 whereas output shaft is connected to gear 6. The motor shaft rotates at 1200rpm in clockwise direction. Find the direction and speed of the output shaft. Gear; 1; 2; 3; 4; 5; 6. No’s of Teeth; 30; 75; 40; 120; 28; 70. | (3) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simple band brake shown in Fig.4 is applied to shaft carrying a flywheel of 3924N. The radius of Gyration of the flywheel is 450mm and runs at 300rpm. If the coefficient of friction is 0.2 and the brake drum diameter is 240mm, find i) Torque applied due to Hand load of 98.1N ii) Number of turns of wheel before it is brought to rest. |  |
| b) | Explain with the help of a neat sketch the construction and working of a rope Brake dynamometer. |  |
| c) | Four masses A, B, C and D are attached to a shaft and revolve in the same plane. The masses are 12Kg, 10Kg, 18Kg and 15 Kg respectively and their radii of rotations are 40mm, 50mm, 60mm and 30mm. The angular positions of the masses B, C and D are 60, 135 & 270 from mass A. Find the magnitude & position of the balancing mass at a radius of 100mm. |  |
|  |  | …….4/- |