## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Electronics & Drive Control(5102)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give three points of distinction between DIAC & TRIAC. |  |
| b) | What precaution is to be taken during series connection of diodes? |  |
| c) | State the three important features of fast recovery diode. |  |
| d) | What is the purpose of Snubber circuits? |  |
| e) | How is it possible to recover the energy trapped with a diode? Draw neat circuit diagram. |  |
| f) | State the advantages of Solid State relay. |  |
| g) | Explain in brief working principle of a Chopper. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a two transistor model of a thyristor & explain its regenerative effect. |  |
| b) | State the methods used for turn on of a thyristor. Which method is called as false triggering method? |  |
| c) | What is meant by commutation of SCR? State its types. What type of commutation is used in a.c. operated thyristor? |  |
| d) | What do you mean by pulse triggering of thyristor? State the methods used for pulse triggering. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In case of single phase full wave converter operating resistive inductive load, draw: i) Supply voltage waveform ii) Output voltage waveform iii) Load current waveform iv) Circuit diagram. |  |
| b) | For a single phase controller of your choice draw its: i) Circuit diagram ii) Input voltage waveform iii) Output voltage waveform iv) Load current waveform. |  |
| c) | In case three phase dual converter draw its: i) Circuit diagram ii) Input voltage waveform iii) Output voltage waveform iv) Load current waveform. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat circuit diagram of a buck switched mode regulator & explain its working in brief. |  |
| b) | Draw the circuit diagram of a single phase SCR series inverter and explain its working in brief, with the help of output waveform. |  |
| c) | How is it possible to generate 16 2/3 Hz frequency from a 50 Hz input frequency? Explain with the help of neat circuit diagram, input & output waveform. |  |
| d) | Classify solid state a.c. switches & explain any one type of your choice with the help of neat circuit diagram & input as well as output waveform. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify single phase drives and explain any one of such drive of your choice with the help of neat circuit diagram. |  |
| b) | Classify three phase drives and explain any one of such drive of your choice with the help of neat circuit diagram. |  |
| c) | How is armature voltage controlled in d.c. motor by using chopper control? Explain with the help of neat circuit diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the methods used for speed control of a.c. drives. What is the significance of voltage to frequency control in a.c. drives? Draw a neat circuit diagram used for voltage to frequency control. |  |
| b) | What is slip power? How can it be recovered? Explain with the help of neat circuit diagram, the method used for slip power recovery scheme. |  |
| c) | How can stator voltage control be applied in a.c. motor control? Explain any one of such method of stator voltage control using thyristorised devices. |  |

***********