## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define and state units of: i) Illuminance ii) Luminance. |  |
| b) | Give classification of luminaires. |  |
| c) | State any six advantages of electrical heating. |  |
| d) | Briefly explain principle of dielectric heating. |  |
| e) | State any four advantages and two applications of infrared heating. |  |
| f) | Briefly explain importance of straight polarity and reverse polarity in arc welding. |  |
| g) | With neat diagram explain projection welding. |  |
| h) | List any six requirement of ideal traction system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain construction and working of core type induction furnace. |  |
| b) | With a neat diagram explain working of mains frequency coreless induction furnace. |  |
| c) | With a neat diagram explain any one indirect resistance heating furnace. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain batch type resistance furnace. |  |
| b) | List properties to be possessed by material required as heating element. Explain any three causes of failure of heating element. |  |
| c) | Calculate the heat developed by a 20 KW nichrome wire for its heating element. The wire temperature should not exceed 1127C & temperature of charge is 427C. Assume emissivity =0.9 and radiant efficiency = 0.6. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between DC welding & AC welding. (any six points). |  |
| b) | List types of electrodes used for welding. Explain advantages of coated electrodes. |  |
| c) | Briefly explain process of arc welding. State its advantages. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain and state application of following systems of track electrification: i) D.C. System ii) Single phase low frequency system. |  |
| b) | With neat diagrams, explain bridge transition method employed for traction motors. |  |
| c) | With neat diagram explain: i) Bow collector ii) Pentograph collector. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain working of fluorescent tube. Also state functions of magnetic ballast and starter. |  |
| b) | With a neat diagram explain High pressure sodium vapour lamp. State its application. |  |
| c) | Briefly explain i) Cut off luminaire ii) Semi cut off luminaire. |  |

***********