## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg./A

Subject: Machine Drawing-I(4033) [Old]

# Time Duration: 4 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw the projections of a line 80mm long inclined at 30 to HP and its top view appears to be inclined at 60 to VP. One of the ends of the line is 45mm above HP and 60mm infront of VP. Draw its projections by the auxiliary plane method. |  |
| b) | A pentagonal plane lamina of edges 20mm is resting on HP with one of its corner touching it such that the plane surface makes an angle of 60 with HP. The two of the edges containing the corner on which the lamina rests make equal inclinations with HP. When the edge opposite to this corner makes an angle of 45 with VP, draw the top and the front views of the plane lamina in this position using Auxiliary Plane method. |  |
| c) | Draw the top and front views of a right circular cylinder of base 45mm diameter and 60mm long when it lies on HP such that its axis is inclined at 30 to HP and the axis appears to be perpendicular to VP in the top view. Draw the projections using change of ground line method. |  |
| Q.No.2. | Answer the following: |  |
| a) | A vertical hexagonal prism of 30mm side of base and axis 65mm long has one of its rectangular faces parallel to VP and nearer to it. A circular hole of 400mm diameter is drilled through the prism completely such that the axis of the hole bisects the axis of the prism at right angles and is perpendicular to VP. Draw the development of the prism showing the shape of the hole on it. | (10) |
|  | ### OR |  |
| b) | A right regular hexagonal pyramid of 30mm side of base and height 70mm stands with its base on HP with one of the base edges parallel to VP. A through circular hole of 30mm diameter is drilled through the pyramid such that the axis of the hole is perpendicular to VP and intersects the axis of the pyramid 20mm above the base. Draw the development of the lateral surface of the pyramid showing the true shapes of the holes formed on it. | (10) ……..2/- |
|  | -2- |  |
| Q.No.3. | Answer the following: |  |
| a) | A vertical triangular prism sides of base 50mm has one of its rectangular faces parallel and nearer to the VP. Draw the curves of intersection when it is penetrated by a horizontal square prism of base edges 20mm long. The two rectangular faces of the square prism make equal inclinations to the HP and its axis is at right angles to the axis of the other prism and 8mm from it. The axis of the square prism is parallel to the VP. | (12) |
|  | ### OR |  |
| b) | A cylinder 40mm diameter is resting on its base on the ground. Another cylinder 35mm base diameter intersects the first cylinder. The axes are parallel to the VP and are inclined at 60 to each other. The axis of the vertical cylinder is 5mm nearer to the VP compared to the axis of the inclined cylinder. Draw the curves of intersection. | (12) |
| Q.No.4. | Fig.1 shows the orthographic views of an object by 1st angle method of projection. Draw using the same angle of projection: | (16) |
|  | i) Sectional Front view ii) Top view iii) Left hand side view. |  |
| Q.No.5. | Answer the following: |  |
| a) | Give the conventional representation for the following: i) Wood ii) Liquids iii) Concrete iv) Metals. | (8) |
|  | ### OR |  |
| a) | With the help of suitable free hand sketches show the following: i) Full section ii) Revolved section iii) Partial section iv) Offset section. | (8) |
| b) | Draw by double line method a 25mm diameter pipe showing: i) 45 Elbow ii) Gate valve iii) Cap. | (9) |
| Q.No.6. | Answer the following: |  |
| a) | Redraw the given views in Fig.2 by a neat proportionate free hand sketch. Plates marked as 1, 2, 3 and 4 are to be connected as mentioned below. Use proper welding symbols to show on the sketch for following joints: i) Piece 1 is connected to Piece 2 by single J butt weld. ii) Piece 2 is connected to piece 3 by all round fillet arc welding. iii) Piece 1 is connected to piece 4 by square butt weld. Finished flush on top. | (8) |
|  | ### OR |  |
| b) | Show the welding symbols for the following types of the welds: i) Square butt weld ii) Convex double V weld iii) Single J butt weld iv) Spot weld v) Fillet weld vi) Seam weld vii) Slot weld viii) Single V butt weld. | (8) |
|  |  | ……3/- |