## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Design of Structures-I(Steel)[4024]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS: 800-1984 and steel tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Draw stress-strain curve for mild steel and explain the salient points on it. |  |
| b) | With neat sketch, explain the design procedure for eccentrically loaded welded connection when load is not lying in the plane of weld. |  |
| c) | What are plate girders? Where are they used? Briefly explain the functions and location of bearing stiffeners in plate girders. |  |
| d) | State the general specifications considered in design of battened columns. |  |
| e) | What are the advantages and disadvantages of trusses with tubular section? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two flats 100 x 10mm and 100 x 12mm are connected by a lap joint under tension. Design the joint using permissible stress in tension equal to 150MPa and permissible stress in weld equal to 108 MPa. |  |
| b) | State advantages and disadvantages of welded joints. Explain with neat sketch i) Intermittent weld and ii) Effective length of a weld. |  |
| c) | Design a welded connection using ISMC 150 as a tension member to develop full strength of the member. Take permissible stress in weld = 150MPa. Assume gusset plate of 10mm thickness. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Tie member in a truss is 1.8m long and is required to carry pull of 180KN. Design using a single unequal angle with longer leg connected. | (7) |
|  | ### OR |  |
| a) | Compare the tensile capacities of 2-ISA 90x90x8mm when connected on i) Same side of gusset plate ii) Either side of gusset plate. Assume gusset plate of 8 mm thickness and members are tack welded in both the above cases. | (7) |
| b) | Determine the tensile capacity of ISNT 75 as a tension member. | (5) |
| Q.No.4. | Answer the following Questions: |  |
| a) | State the codal provisions for maximum slenderness ratio. | (4) |
|  | ### OR |  |
| a) | Briefly explain the design procedure for slab base. | (4) |
| b) | Design a single angle strut for a roof truss to carry a compressive load of 125KN. Take effective length of member as 2m. Design the welded connection also. | (8) |

………2/-

-2-

| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain how deflection check is given for simply supported beams carrying u.d.l. If the deflection check is not satisfied, how would you design the section to get deflection check satisfied. |  |
| b) | Design a floor beam for a hall admeasuring 16.0x7.5m. Rolled steel joists are placed at 4m c/c. Joists support RCC slab of 14cm thickness. Assume wall of 30cm thick all around. Take live load of 4 KN/m2. Check for deflection and shear. |  |
|  | ### OR |  |
| b) | ISMB 500 is to be used as a beam over simply supported span of 6.0m. Beam supports two equidistant point loads of ‘W’ each. Find the point load W. Also, check for shear. Assume permissible bending stress of 165 MPa. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Explain the procedure for design of angle section purlins. | (4) |
|  | ### OR |  |
| a) | Explain how internal and external wind pressure on trusses are calculated. | (4) |
| b) | Calculate the dead load and live load on each panel point for the following data: - Span of the truss = 15.30m - Spacing of purlins = 1.45m - Spacing of trusses = 4.5m - Slope of the truss = 28.4 - Weight of AC sheet roofing = 175 N/m2 | (8) |

***********