## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./E/EI/COMP/EC/MED/E&EE

Subject: Digital Electronics-I(4134) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert the fall into binary: (531)10, (5D)16, (123)8 |  |
| b) | Draw the logic diagram and write the truth table of SR flip flop. |  |
| c) | Prove Duality theorem with suitable example. |  |
| d) | Compare combinational and sequential digital system. |  |
| e) | Compare synchronous and asynchronous counter. |  |
| f) | Draw the logic diagram and write the truth table of NAND, OR and NOT gate. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prove NOR as a universal gate. |  |
| b) | Prove the following expression using Boolean laws: i) (A+B)(A+B)(A+C)=AC ii) AB+AB+AB=A+B |  |
| c) | Add the following decimal numbers using 8-bit 2’s complement arithmetic: (i)+60, -28 (ii)+50, -21 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Simplify the following Boolean expression and draw using NAND gates. F (A, B, C, D) = (0,2,5,7,8,10,12,13,14,15) |  |
| b) | Explain the working of edge triggered D-Flip flop with relevant diagram. |  |
| c) | For the following flip flops draw the logic diagram and also write its truth table: (i) T flip flop (ii) D flip flop. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a 4-bit serial in parallel out shift register. |  |
| b) | Explain the operation of 3-bit up–down counter with necessary timing diagram and logic diagram. |  |
| c) | Draw a synchronous mod-3 counter and explain its working. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the working of (i) DCTL-NOR gate (ii) RTL-NOR gate. |  |
| b) | Draw the basic circuit of ECL logic family and explain its working. |  |
| c) | Explain the various characteristics of digital logic families. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Tristate Logic |  |
| b) | High speed TTL |  |
| c) | PIPO Register |  |
| d) | Race around condition |  |
| e) | Clocked SR flip flop |  |

***********