## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Architectural Engg.

Subject: Building Construction-IV (AE402) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the advantage of using welding as compared to riveted connections in steelwork? |  |
| b) | What does the arrangement and sizes of various members of a steel roof truss depend upon? |  |
| c) | Why is the position of D.P.C. important before locating it in a building? |  |
| d) | Give examples of components in a building which require waterproofing. |  |
| e) | Give any three objectives of plastering. |  |
| f) | What should the structural design of upper floor ensure? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give any six points which need to be considered while designing and construction of a steel roof truss. |  |
| b) | Mention at least six advantages of using a steel truss as compared to a timber truss. |  |
| c) | Explain with a sketch the steel work fabrication procedure by using riveting. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following methods adopted in making a building damp proof (i) Pressure grouting (ii) use of D.P.C. |  |
| b) | Describe with a neat sketch the method of damp proofing for foundation on damp soil. |  |
| c) | Explain different types of fabrication procedures for steel structures. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between the waterproofing technique of an overhead water tank and an underground water sump. |  |
| b) | Explain with a neat cross-sectional sketch, the waterproofing treatment given to a bathroom. |  |
| c) | Explain with examples the types of welding joints in steelwork. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the method of colourwashing for an existing old building under renovation. |  |
| b) | Describe any three different kinds of plaster finishes. |  |
| c) | Draw the steel joinery details of a steel king post truss. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat plan and section the construction details of a double joists timber floor. |  |
| b) | Explain with a neat sketch the cross section of a reinforced brick floor. |  |
| c) | Draw a neat sketch showing the hollow tile flooring and state its advantages. |  |

***********