## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Maintenance Engineering(5035)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the requirements of lubricant. |  |
| b) | What are the precautions to be taken while mounting a bearing on the shaft? |  |
| c) | How is a broken key removed and replaced? |  |
| d) | Differentiate between predictive and preventive maintenance. |  |
| e) | State the advantages and disadvantages of time study in maintenance work. |  |
| f) | Write a short note on servicing of pneumatic valves. |  |
| g) | State the use of Vernier calliper and Micrometer in maintenance work. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How is the planning carried out using PERT and CPM technique during shutdown maintenance of an industrial setup? |  |
| b) | What is maintenance estimation? How would you classify the maintenance job? |  |
| c) | Briefly explain the steps to be followed for organising predictive maintenance programme. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the causes of machine vibration. |  |
| b) | Explain ‘Baseline’, ‘warning’ and ‘danger limits’ of vibration with respect to condition monitoring. |  |
| c) | What do you mean by standard time for maintenance work? Explain with a simple example how would you calculate standard time for a piece of maintenance work. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How can the cost of inventory related to maintenance be minimized? Discuss. |  |
| b) | Describe the advantages of centralized maintenance storeroom. |  |
| c) | Discuss with an example the use of welding and grinding methods for reconditioning of components. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief the content of maintenance manual. |  |
| b) | What are the common defects found in valves? How is the gland packing replaced in valves? |  |
| c) | With help of simple sketches, explain the various modes of lubrication. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Tomilson’s surface meter b) Bend Protactor |  |
| c) | Failure of journal bearing d) Cleaning of filter |  |
| e) | Use of greasing gun f) On-line maintenance |  |

***********