## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Process-I(4233)/(FE303) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between Line standard and End standard. |  |
| b) | What do you mean by physical measurement? |  |
| c) | How can compasses be used in marking? |  |
| d) | Write a short note on pipe square. |  |
| e) | State the advantages and disadvantages of abrasive cutting machine. |  |
| f) | State the factors influencing the quality of cut. |  |
| g) | State the procedure for lighting the cutting torch. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of a neat sketch the construction of Dial Indicator. |  |
| b) | Describe radius gauges with a neat sketch. |  |
| c) | Explain the use of Engineer’s Square in squareness testing with a neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the method of marking out bolt holes for flanges with a neat sketch. |  |
| b) | State the procedure for marking of holes in ‘Tee’ sections. |  |
| c) | How are templates used as a means of checking? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a diagram explain the following terms: i) Tolerance ii) Nominal size iii) Zero line iv) Limits of size |  |
| b) | Explain the different types of Limit Gauges. |  |
| c) | Explain the principle of micrometer screw gauges. State the uses of Vernier height gauge. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the basic principle of flame cutting. |  |
| b) | State the safety precautions required for metal sawing. |  |
| c) | Compare flame cutting with shearing. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Use of water line |  |
| b) | Wire and thickness gauge |  |
| c) | Use of beam trammels |  |
| d) | Protection and storage of templates |  |
| e) | Possible error when using sriber |  |
| f) | Reciprocating power hacksaw |  |

***********