## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Principles of Refrigeration(5046)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between Heat pump and refrigerator. |  |
| b) | Define: i) Tonne of refrigeration ii) Coefficient of performance. |  |
| c) | In a vapour compression refrigeration system, the refrigerant enters the compressor in ‘wet’ state and leaves in superheated condition. Draw P-H and T-S diagram for the above cycle. |  |
| d) | Explain in brief the effect of CFC on ozone layer. |  |
| e) | State four points of difference between Hermetically sealed compressor and an open type compressor. |  |
| f) | What are the functions of refrigeration controls? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State at least two functions of each of the following components in a refrigeration system: i) Compressor ii) Condensor iii) Throttling device iv) Evaporator. |  |
| b) | With the help of P-H diagram, explain the effects of following on refrigeration systems: i) Increase in condensor pressure ii) Decrease in Evaporator pressure. |  |
| c) | In a R-12 refrigeration system, the condensation and evaporation takes place at 50C and 15C respectively. The temperature at the end of Isentropic compression is 56C. Assuming simple saturation cycle, determine the following: i) Theoretical COP ii) Carnot C.O.P. iii) Refrigeration capacity if 0.2 Kg of refrigerant is circulated in one second. Use the properties of R-12 given below. |  |
|  | Saturation Temperature in C; Enthalpy in KJ/Kg. Liquid (hf); Vapour (hg). 15; -; 193.8. 50; 84.9; 206.3. Specific heat of vapour Cpv = 0.8 KJ/Kg -K |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working principle of centrifugal compressor used in refrigeration system. |  |
| b) | With a neat sketch explain the construction and working principle of ‘Evaporation type condensor’. |  |
| c) | With a neat sketch explain the construction and working of ‘capillary tube’. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the chemical formula and name of the following refrigerants: i) R-152 ii) R-134. |  |
| b) | Write a short note on following properties of refrigerants: i) Action with lube oil ii) Action with material of construction and stored food. | …….2/- |
|  | -2- |  |
| c) | State at least three advantages and three limitations of vapour absorption refrigeration system as compared with vapour compression system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain ‘Electrolux’ refrigerator. |  |
| b) | With a neat sketch explain the construction and working principle of low pressure cut-out. |  |
| c) | What are the probable causes and effects of high pressure on the refrigeration system? |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a neat labelled block diagram of ‘Ice Plant’. |  |
| b) | Sketch a neat labelled block diagram of ‘cold storage plant’. |  |
| c) | Write short note on role of refrigeration system in ‘Breweries’. |  |

***********