## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Engineering & Technology

Subject: Engineering Mechanics(CS301) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between Resultant and Equilibrant force. |  |
| b) | List down the different types of supports in beams. |  |
| c) | Draw F.B.D. for a rectangular block resting on a rough inclined plane subjected to a force parallel to the plane. |  |
| d) | State D’ Alemberts principle. |  |
| e) | Explain cone of friction. |  |
| f) | State the law of conservation of momentum. |  |
| g) | Explain what is Kinetic and Potential energy. |  |
| h) | What is a couple? Give any three characteristics of a couple. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Five wires are attached to a vertical post and they exert the following pull on the post: 1) 80N due South 2) 150N South West 3) 100N due East 4) 90N North West 5) 120N due North. Find the resultant pull on the post. |  |
| b) | Find the resultant in magnitude and direction of the force system shown in fig.(1). |  |
| c) | Determine the resultant of the given system of forces shown in fig.(2). |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the reaction at the surface of contact at point A and point B of a smooth cylinder lying in triangular groove. Ref. Fig.(3). |  |
| b) | Determine the reactions at supports for the beam subjected to loading as shown in fig.(4). |  |
| c) | Determine the forces in all the members of a cantilever truss shown in fig.(5) by method of joints. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A pull of 70N inclined at 25 to the horizontal plane, is required just to move the body placed on a rough horizontal plane. But a push required to move the body is 80N. If the push is inclined at 25 to the horizontal, find the weight of the body and the coefficient of friction. |  |
| b) | A uniform ladder of weight 900N and of length 7m rests on a horizontal ground and leans against a smooth vertical wall. The angle made by the ladder with the horizontal is 60. When a man of weight 600N stands on the ladder at a distance of 4m from the top of ladder, the ladder is at the point of sliding. Determine the coefficient of friction between ladder and the floor. |  |
| c) | Four forces equal to P, 2P, 3P and 4P are respectively along the four sides of a square ABCD taken in order. Find the magnitude, direction and position of the resultant force. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An elevator of mass 1960Kg is to be lifted and lowered by means of a rope. Find the tension in the rope when the elevator is moving upwards with acceleration of 2m/sec2 and moving downwards with acceleration of 1.5m/sec2. |  |
| b) | Two rough planes are inclined at 30 and 60 to the horizontal having a smooth pulley at the apex of the planes. Masses 5Kg and 10Kg are placed on the planes as shown in fig.(6). If the coefficient of friction is 13, find the resulting acceleration and tension in the string. |  |
| c) | A wagon of mass 10 tonnes rolling at 1.5 m/sec collides with another wagon of mass 15 tonnes and rolling at 2 m/sec in the opposite direction of the same track. After collision the two wagons move together. Find the magnitude and direction of their common velocities. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A chain weighing 90N/m is 60m long. It hangs vertically. A body of mass 500Kg is attached at the lower end of this chain. Find the work done in raising this body through 60m by winding the chain. Find also the work done for complete winding of the chain. |  |
| b) | A pump reduces the water level in a wall 3m diameter by 1.5m in 20 minutes and lifts the water through an average height of 4m. If the efficiency of the pump is 80%, what power is required to drive the pump? |  |
| c) | A body of mass 4 Kg drops freely from a height of 50m and penetrates into the ground by 1m. Find the average resistance of penetration. |  |