## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(4339)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Froude’s number |  |
| b) | Dented propellers |  |
| c) | Air resistance |  |
| d) | Thrust deduction fraction |  |
| e) | A-bracket |  |
| f) | Pitch of the propeller |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Explain frictional resistance, wall making resistance and eddy resistance. | (9) |
| b) | Define break horsepower and mechanical efficiency of engine. At what point on the engine, break horsepower is measured. | (3) |
| Q.No.3. | Answer the following Questions: |  |
| a) | A model of a ship is towed in freshwater. The length of the model is 5.2m, wetted surface area is 4.25m2 and speed of towing is 1.8 m/sec. The ship is of length 131m. Determine the total resistance of the ship. Kinematic viscocity = 1.14x10-6 m2/sec FW Kinematic viscocity = 1.19x10-6 m2/sec SW | (8) |
| b) | Briefly explain how reversing is obtained in a typical gearbox. | (4) |
| Q.No.4. | Answer the following Questions: |  |
| a) | With neat sketch explain following propeller terms: i) Projected area of the propeller ii) Developed area of the propeller iii) Propeller blade section. | 2 + 2 + 4 |
| b) | Sketch and describe contra-rotating propellers. | (4) |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain estimation of resistance of the ship by Guldammer’s and Harvard’s method. |  |
| b) | Sketch stern gear arrangement of ship and briefly describe shaft bearing and thrust block. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | A propeller is having pitch of 5.5m and speed of 3.5 rev/sec. The delivered power is given as 2,300 KW and propeller efficiency of 70%. If the speed of the ship is 13.2 Knots and the speed of advance is 11 Knots, find i) Real slip ii) Wake fraction iii) Propeller thrust. | (7) |
| b) | Sketch and describe controlled pitch propeller. | (5) |
| Q.No.7. | Answer the following Questions: |  |
| a) | Sketch and describe water-cooled stern tube. | (8) |
| b) | What is the use of reversing action of a gear box in propulsion system installed with controlled pitch propeller? | (4) |

***********