## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(5036)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat sketch showing the power flow from the engine to the wheels in a 2 WD vehicle. |  |
| b) | What is the function of drier in the car air conditioning system? |  |
| c) | State one possible cause of ‘hard gear shifting’ and necessary remedy for the same. |  |
| d) | What is meant by ‘stroboscopic effect’? What is the use of stroboscopic light in automotive maintenance? |  |
| e) | Explain how wind resistance varies with vehicle speed. |  |
| f) | How is a semi-elliptic leaf spring a beam of uniform strength? |  |
| g) | Define camber angle and state the reason why it is provided. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain conventional construction of an automobile chassis. |  |
| b) | With the help of a neat sketch, explain the construction and working of wish bone type of independent suspension. |  |
| c) | With the help of a neat sketch, explain the steering linkage of a rigid axle suspension vehicle. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between: i) MPFI and TBI ii) Timed and Continuous injection. |  |
| b) | Explain the engine management system of an MPFI engine. |  |
| c) | Explain the procedure of adjusting ignition timing of a petrol engine by adjusting the distributor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of a sliding mesh gear box. |  |
| b) | Distinguish between fluid flywheel and torque converter. |  |
| c) | What is ‘double declutching’? Why is it done? Is it required to be done in synchromesh gear box? Why? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe three ways of rating a battery. |  |
| b) | Explain battery-and-coil ignition system with the help of a neat circuit diagram. |  |
| c) | With the help of a neat sketch, explain the working of the cut out relay in the charging system in an automobile. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat labelled diagram explain the layout of the refrigeration system for car air conditioning. |  |
| b) | State the provisions under section 115 of Motor Vehicle Act 1988. |  |
| c) | What is the use of a torque wrench? Draw neat sketches of various types of torque wrenches. |  |

***********