## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Shipbuilding Engg.

Subject: Shipdrawing & Calculations(4346)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data, if necessary.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | KN curves of stability |  |
| b) | Subdivision factor |  |
| c) | Types of Bulkheads |  |
| d) | Floodable length curves |  |
| Q.No.2. | The following information has been obtained from the cross curves of stability of a ship of displacement 40,000 tonnes and KG=10 m. | (12) |
| Angle of heel; 0; 5; 10; 15; 20; 30; 45; 60; 75; 90. KN(m); 0; 0.9; 1.92; 3.11; 4.25; 6.3; 8.44; 9.39; 9.29; 8.5. |
|  | Determine i) Metacentric height ii) Dynamic stability at 15. |  |
| Q.No.3. | How is the scantling calculations of plating and stiffners for bulkhead carried out? Why is the bulkhead plating thickness more at bottom and less as we go higher? | (12) |
| Q.No.4. | Explain in detail the significance of subdivision with neat sketches. Define the term ‘Floodable length’. How are floodable length curves used to determine the spacing between the bulkheads in passenger ships? | (12) |
| Q.No.5. | A ship of 15000 tonne displacement has righting levers of 0, 0.38, 1.0, 1.41 and 1.2 m at angles of heels of 0, 15, 30, 45 and 60 respectively and assumed KG of 7m. The vessel is loaded to this displacement but the KG is found to be 6.80m and GM = 1.50m. - Draw the amended stability curve. - Estimate dynamic stability at 60. | (12) |
| Q.No.6. | Starting from the bonjean curves of a vessel, explain in detail how the floodable length curve of a vessel is determined. | (12) |
| Q.No.7. | A vessel is loaded up ready for departure. KM is 11.9m, KG is 9.52m with a displacement of 20550 tonnes. From the ships cross curves of stability, the GZ ordinates for a displacement of 20,550 tonnes and a VCG of 8m above base are as follows: | (12) |
| Angle of heel; 0; 15; 30; 45; 60; 75; 90. GZ ordinate; 0; 1.10; 2.22; 2.60; 2.21; 1.25; 0.36. |
|  | Using this information construct the stability curve and find: i) Maximum righting lever GZ ii) Range of stability iii) Angle of heel at which maximum GZ occurs. |  |

***********