## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Automobile Engg.

Subject: Hydraulic Machinery(4053)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is a Piezometer tube? What are its drawbacks? |  |
| b) | Find the pressure at a point in a sea 50m below the free surface. Take specific gravity of sea water as 1.1 |  |
| c) | Define total pressure and centre of pressure for a surface immersed in liquid. |  |
| d) | State Bernoullis theorem. What are the assumptions made for the theorem? |  |
| e) | State the laws of fluid friction. |  |
| f) | Explain the following terms with reference to centrifugal pump i) Static head ii) Manometric head. |  |
| g) | Define slip and negative slip in a reciprocating pump. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular plate 4m long and 1.5m wide is immersed vertically in water in such a way that its shorter side is parallel to the water surface and is 2m below it. Find i) Total pressure on the plate ii) Position of centre of pressure. |  |
| b) | A 300mm x 100mm venturimeter is provided in a vertical pipe carrying water, flowing in the upward direction. A differential mercury manometer connected to the inlet and throat gives a reading of 220mm. Find the rate of flow. Assume Cd = 0.98. |  |
| c) | With the help of a neat sketch, explain the working of inverted manometer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the hydraulic coefficients of an orifice. Explain vena contracta with a neat sketch. |  |
| b) | A rectangular notch has a discharge of 250 litres/s when the head of water is 1m. Find the length of the notch if Cd = 0.6. |  |
| c) | What is an air vessel? Explain its various functions in reciprocating pumps. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain hydraulic gradient line and total energy line for a pipe connecting two reservoirs. |  |
| b) | With the help of neat sketches explain the three types of casings used in centrifugal pump with their applications. |  |
| c) | Why centrifugal pump needs priming? Explain the different methods of priming. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of a neat sketch explain the construction and working of a plunger pump. |  |
| b) | With the help of a neat sketch, explain the construction and working of a hydraulic accumulator. |  |
| c) | Compare reciprocating pump with centrifugal pump with respect to construction, working and applications. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a neat labelled diagram of a gear pump. State its one application. |  |
| b) | Find the head lost due to sudden contraction in a horizontal pipe which reduces from 200mm to 150mm in diameter. Water is flowing through it at a rate of 0.05 m3/s. |  |
| c) | With the help of a simple sketch explain the working principle of a hydraulic press. |  |
| d) | Define i) Viscosity ii) Specific weight. |  |

***********