## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT

### Subject: Design of Structures-II(RCC)[4016]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 456-2000 is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe the difference in behaviour of one-way slab and two-way slab. |  |
| b) | Define limit state method. What are the objectives of limit state method? |  |
| c) | What is meant by ‘Limit state of serviceability’? |  |
| d) | Explain truss analogy for shear design. |  |
| e) | Define Bond stress. State its significance. |  |
| f) | What are the precautions to be taken at design, detailing and construction stages to ensure high qualify structure? |  |
| g) | Enlist the various loads to be accounted for design and the load combinations. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the moment of resistance of a section 300mm wide and 450mm deep upto the center of reinforcement. It is reinforced with 4 Tor 12mm. Use M20 grade concrete and fe415 steel. |  |
| b) | A simply supported R.C. beam 230mm wide & 450mm effective depth is provided with 4 Tor 20mm bars tensile reinforcement. If the effective span of the beam is 4.5m. Determine working load at mid-span. The grade of concrete is M20 and fe415 steel. |  |
| c) | A singly reinforced beam 230mm wide and 400mm deep is reinforced with 4 bars of 16mm diameter with 40mm effective cover. State whether the beam is balanced, under-reinforced or over-reinforced. Also determine the ultimate moment capacity of the beam. M20 & fe500 are the grades of concrete & steel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the moment of resistance of a beam 300mm wide and 500mm deep if it is reinforced with 2 Tor 12mm bars in compression zone and 4 Tor 20mm bars in tension zone, each at an effective cover of 40mm. Assume M20 concrete & fe500 steel |  |
| b) | Design a doubly reinforced beam for effective span 5.5m to carry service load of 45 KN/m including self weight. Assume concrete grade of M20 and fe500 grade steel. Restrict beam to size of 230mm x 450mm. |  |
| c) | A simply supported beam of size 230mm x 450mm is reinforced with 4 bars of 12mm . Assume M20 grade concrete and fe415 steel. Design shear reinforcement for the beam. The beam is subjected to a factored shear of 90KN at the support. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine tension and compression reinforcement for an R.C. beam, for the following data. Span =5m, uniformly distributed factored load (including self weight) = 75KN/m., width of beam restricted to 230mm, depth of beam restricted to 500mm (overall), Effective cover on both sides is 40mm. Use M20 concrete & fe415 steel. |  |
| b) | A R.C.C. beam 250mm wide and 450mm deep is reinforced with 3 Tor 20mm bars of grade fe415, on the tension side with an effective cover of 50mm. If the shear reinforcement of 2 legged 8mm stirrups at a spacing of 160mm c/c is provided, determine design (ultimate strength) of the section. Use M20 concrete. |  |
| c) | Sketch details of reinforcement at i) Offset column ii) Members with a change in direction. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate reinforcement required for a chajja of effective span 0.7m and design load of 7KN/m. Assume M20 grade concrete and fe415 steel. |  |
| b) | Design a cantilever slab for an effective span of 1.65m with live load 2.5 KN/m2 and finishing load of 1KN/m2. Assume concrete grade M20 and fe415 steel. |  |
| c) | Design a slab for a corridor 2m wide and 7m long. 230mm wide beam supports all around the slab. Take M20 concrete and fe415 steel. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A short column with 300mm x 300mm is reinforced with 4 Tor 16mm bars as longitudinal reinforcement, Tor 8mm lateral ties are provided at 200mm pitch. If M20 grade of concrete and fe415 steel is used, determine the safe axial load at service condition. |  |
| b) | Design a rectangular column (B/D =1.5) to carry an axial load of 1000 KN. The effective length of column is 3.2m. Adopt M20 concrete and fe415 steel. |  |
| c) | Design a square column footing to carry a column load of 800 KN from 400mm x 400mm square tied column consisting 20mm bars as the longitudinal steel. The bearing capacity of soil is 100KN/m2. Density of earth is 20 KN/m3. Use M20 concrete and fe500 steel. |  |

***********