## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Engineering & Technology

Subject: Applied Mechanics-I(2005) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Differentiate between vectors and scalars. |  |
| b) | State and explain principle of transmissibility with an example. |  |
| c) | State characteristics of a couple. |  |
| d) | Define coefficient of friction and angle of friction. State the relationship between the two. |  |
| e) | What are the conditions of equilibrium for non concurrent force system? |  |
| f) | State and explain law of parallelogram of forces. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Force of magnitude 20N, 30N, 40N, 50N, 60N act at one of the angular points of a regular hexagon towards the other five angular points. Find the resultant in magnitude and direction. |  |
| b) | A parallel force system acting vertically has following forces: |  |
|  | i) 50N upwards ii) 100N downwards iii) 200N downwards and 500N upwards. The forces are placed at 2m distance between each force. Find the resultant in magnitude, direction and position from first force. |  |
| c) | ABCD is a rectangle of 5cm x 4cm. 40N, 60N, 80N and 100N are force acting along the sides AB, BC, CD and DA taken in order. Find the resultant in magnitude, direction and position from A. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A rope is connected between two points A and B 120 cm apart at the same level. A load of 1000N is suspended at C on the rope as shown in fig.3(a). Also another weight W is suspended at D. Find the unknown weight and tension in the strings AC, CD, DB. |  |
| b) | A simply supported beam AB of span 6m is loaded as shown in fig.3(b). Find the reactions at A & B. |  |
| c) | A beam ABCD hinged at A and supported on rollers at D is loaded as shown in fig.3(c). Find the reactions at A and D. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A body, resting on a rough horizontal plane required a pull of 180N inclined at 30 to the plane just to move it. It was found that a push of 220N inclined at 30 to the plane just removed the body. Determine the weight of the body and the coefficient of friction. |  |
| b) | A square threaded screw jack of mean diameter 50 mm has 3 helix angle of the thread and coefficient of friction 0.06. Find the effort required at the end of handle 450 mm long to raise a load of 20KN. |  |
|  |  | ……..2/- |
|  | -2- |  |
| c) | A ladder of length 13m and weighing 250N is placed against a smooth vertical wall with its lower end 5 m from the wall. The coefficient of friction between the ladder and the floor is 0.3. Show that the ladder will remain in equilibrium in this position. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A motor car takes 10 seconds to cover 30 metres and 12 seconds to cover 42 metres. Find the uniform acceleration of the car and its velocity at the end of 15 seconds. |  |
| b) | An electric train, travelling between two stations 15 Km apart is uniformly accelerated for the first 10 seconds, during which period it covers 100m. It then runs with a constant velocity until it is finally retarded uniformly in the last 50m. Find the maximum velocity of the train and time taken to complete the journey between the two stations. Use V-T diagram. |  |
| c) | A stone is dropped from the top of a tower, 50m high. At the same time another stone is thrown upwards from the foot of the tower with a velocity of 25 m/sec. When and where the two stones cross each other. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | An I section has the following dimension in mm. Bottom flange 300 x 100, Top flange = 150 x 50, Web= 400 x 50. Determine mathematically the position of the centroid of the section. Ref. fig.6(a). |  |
| b) | A body consisting of a cone and hemisphere of the radius r = 10cm fixed on the same base rests on a table, the hemisphere being in contact with the table. Find the centre of gravity if the height the cone is 18 cm. Ref. fig.6(b) |  |