## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Welding Technology-I(4237)/(FE301) [Old/Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the application of welding in repair and maintenance. |  |
| b) | What is Edge preparation of a weld joint? Explain its importance. |  |
| c) | Explain the need of Flash-Back Arrestor in Acetylene Manifolding system. |  |
| d) | Explain the different types of Free Flight Metal Transfer in Arc welding. |  |
| e) | Mention any three important differences between Brazing and Braze welding. |  |
| f) | What is surfacing? What are its applications? |  |
| g) | Explain, why aluminium and its alloys are more difficult to weld than structural steel, even though its melting point is lower. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define weld slope and weld rotation. Explain the different welding positions based on weld slope and weld rotation. |  |
| b) | Explain with a neat sketch the working of a two-stage gas regulator. |  |
| c) | Differentiate between leftward and rightward techniques of flame welding. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the safety recommendations for Gas cylinders and Pressure regulators. (6 points each) |  |
| b) | What is Arc Blow? Explain its causes, effects and remedies. |  |
| c) | What are arc welding power sources? Why are they required? List the different types of A.C. and D.C. arc welding power sources with a neat chart. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different causes and remedies of porosity in welds. |  |
| b) | Differentiate between Brazing and Soldering. Explain silver Brazing process. |  |
| c) | Explain with a neat sketch Electric Arc Metal Spraying process. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following and also give their need in welding: i) Pre-heating ii) Inter-pass heating iii) Post heating iv) Post weld heat treatments. |  |
| b) | Explain carbon equivalent. How does it affect welding of Alloy steels? |  |
| c) | Mention the type of polarity used in arc welding of Aluminium and their Alloys. Explain its reason. | …..2/- |
|  | -2- |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Oxy-hydrogen gas welding |  |
| b) | Ventilation and health protection |  |
| c) | Importance of flux in soldering |  |
| d) | Furnace Brazing |  |
| e) | AWS-ASTM system of coding of MMAW electrodes |  |

***********