## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Machines-II (EE401) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A 3-Phase, 6-pole induction motor is supplied from a 50Hz, 400V supply. Calculate i) synchronous speed ii) the slip of the motor when running at 960rpm. |  |
| b) | Explain the need of using starter while starting large 3-phase induction motor. |  |
| c) | What are the advantages of rotating field and stationary armature system in case of Alternator? |  |
| d) | What is the need of parallel operation of alternators? |  |
| e) | Explain the concept of synchronous condenser. |  |
| f) | Explain the principle of operation of a repulsion motor. |  |
| g) | Give the relation between i) Starting torque and full load torque and ii) starting torque and maximum torque. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the torque slip characteristics of 3-phase induction motor. What is the effect of variation of rotor circuit resistance on it? |  |
| b) | The input power to a 3-phase induction motor is 50KW and the corresponding stator losses are 2KW. Calculate: - The total mechanical power developed and the rotor I2R loss when the slip is 3 percent. - The output horse power if the friction and windage losses are 1.0 KW and - Efficiency of the motor. |  |
| c) | - Define starting torque, running torque and pull out torque of a 3-phase induction motor. - Draw and explain power flow diagram of a 3-phase induction motor. | (3) (3) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the two types of rotor construction of a 3-phase induction motor. |  |
| b) | Upto what HP rating induction motor can be started with star-delta starter. Hence, explain the manual star-delta starter. |  |
| c) | With a neat circuit diagram, explain the rotor resistance starter for starting slip ring type induction motor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following: - Operation of synchronous generator on NO load. - Operation of synchronous generator on load with unity power factor. | (3) (3) |
| b) | Draw and explain the phasor diagram of an alternator on No load and loaded conditions with lagging power factor load. |  |
| c) | A 2000KVA, 6600V, 3-phase star connected synchronous generator has a resistance of 0.4 ohms per phase and a synchronous reactance of 4.5 ohms per phase. Calculate the voltage regulation at full load, 0.8 p.f. lagging. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working principle of a permanent split phase capacitor start and run single phase induction motor. State its applications . |  |
| b) | Explain the working of working principle of single phase induction motor with the help of double revolving field theory. |  |
| c) | - Calculate the distribution factor for a 36 slots, 4-pole, single layer three phase winding of an alternator. - Give the emf equation of an alternator. Explain the factors on which it depends. | (3) (3) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What conditions are necessary to be satisfied for parallel operations of 3-phase alternators? List the different methods of synchronisation. Explain any one method in detail. |  |
| b) | Explain the effect of change of excitation of a synchronous motor driving a constant load on load current and power factor. |  |
| c) | Explain working of Hysteresis motor. State its applications. |  |

***********