## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Architectural Engg.

Subject: Building Services-I(AE306) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain elevated reservoirs. |  |
| b) | Name and sketch four electrical fittings and fixtures. |  |
| c) | What is storm water drainage? |  |
| d) | Sketch and name any three fittings that are used in water supply. |  |
| e) | Explain in brief different types of traps. |  |
| f) | Write on gradient to be used for soil and waste water drainage. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write in detail the construction and working of a septic tank and soak pit with the help of neat sketch. (Plan & cross-section) |  |
| b) | What are the different types of tests to be conducted for testing of drains before occupying the house? |  |
| c) | Draw sketches of any three of the following: i) Cross-section of an Indian W.C. pan ii) Flushing cistern iii) Types of traps iv) Disconnecting chamber |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw an electrical layout in the given plan. Draw the legend showing fitting and fixtures that are used in any electrical wiring. Refer figure 1. |  |
| b) | What are the different types of wiring? Which is the cheapest and why? |  |
| c) | What are the advantages of the following: i) Ceiling light point and wall light points ii) One way switch and two way switch iii) 5 Amp plug point and 15 Amp plug point. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different types of water supply? Which types of water supply would you recommend for a high-rise building? |  |
| b) | Draw a sanitary layout upto the septic tank in the given plan. Give types of pipes used, diameter of pipe and gradient of slope. Refer figure 1. |  |
| c) | What are the different types of pipes and sizes that are used in a domestic water supply pipe line? Draw any five fitting that are used in a water pipe line. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a cross section of an over head water tank and elevated water reservoir (tank). |  |
|  |  | …2/- |
|  | -2- |  |
| b) | In a multistoried building you have to recommend size of the sump for 100 persons. Calculate the capacity of sump. Assume length and breath and calculate the depth of the sump. What will be capacity of the sump i.e volume and quantity of water? |  |
| c) | Explain different types of sources of water that can be used for consumption directly and which are required to be treated before it is consumed by a human being. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw sketch of the following: i) Tee-joint ii) Reducer iii) Elbow joint. |  |
| b) | Differentiate between conduit wiring and concealed wiring. |  |
| c) | Draw a toilet plan showing W.C. pan, bathrooms area wash basin and nahni trap. |  |
| d) | Differentiate between waste water drainage and night soil drainage. | ….3/- |