## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(4297)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is methane layering? How is it controlled? |  |
| b) | Mention the objectives of Mine Ventilation. |  |
| c) | Differentiate between essential and descential ventilation. |  |
| d) | Write short notes on rigid ducts. |  |
| e) | List any five reasons for creation of turbulent air flow in underground mine. |  |
| f) | What is the purpose of ventilation survey? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the physical, chemical and physiological properties of carbon monoxide. |  |
| b) | What are the sources of heat in underground coal mine? How does humidity & heat affect an underground worker? |  |
| c) | Air temperatures in a downcast shaft 465 meters deep is 30C and that in upcast shaft of same depth is 37C. What assistance, expressed in H.P, does this difference in temperature render when quantity of air passing through the shaft is 100m3/sec? Assume average barometric pressure in DC shaft to be 750mm of Hg. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The velocity of air in a 6m dia O.C shaft 500m deep is 4.5 m/sec. Calculate the quantity of air entering the mine per minute. Also calculate the H.P. lost to overcome the frictional resistance of the shaft. Assume coefficient of friction to be 1.61 x 10-3 mm w.g for a velocity of 1m/s. |  |
| b) | Explain the purpose & construction/erection of ventilation door. How air lock is created & how it helps? |  |
| c) | What is splitting? Give its advantages & disadvantages. What would be the equivalent resistance of two galleries of resistance “R” each connected in series? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the term equivalent orifice. Total quantity of air passing through a district of a mine is 25m3/sec with a w.g. of 20mm. It is now proposed to reduce the quantity of air flowing through the district to 20m3/sec. Calculate the size of the regulator that should be fitted in the district. |  |
| b) | Explain the construction & working of axial flow fan with a neat sketch. |  |
| c) | A 5m diameter centrifugal fan with forward blades delivers 9000m3/min of air when running at 300 rpm. If the blade angle is 35 and width of fan at periphery is 1.4m, calculate the theoritical w.g. developed by the fan. If the actual w.g. obtained is 470 mm, calculate the manometric efficiency of the fan. Assume the density of air to be 1.2 Kg/m2. | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the ventilation of narrow heading by overlapping & reversal methods. |  |
| b) | Explain the method of performing quantity survey. |  |
| c) | i) Draw a neat sketch of characteristic curves of any fan. ii) How do you test for methane by using methanometer? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the principle behind the working of a Flame Safety lamp. |  |
| b) | What are the factors affecting NVP? |  |
| c) | Differentiate between forcing & exhaust ventilation. |  |
| d) | Pitot tube is used to find out velocity pressure. Comment. |  |

***********