## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./EC/EI/MED

Subject: Analog Electronics(EN405) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the equivalent circuit of an Op-amp. |  |
| b) | Draw the block diagram of: i) Voltage series feedback amplifier ii) Voltage shunt feedback amplifier. |  |
| c) | Draw the circuit of a grounded load voltage to current converter. |  |
| d) | List 3 advantages of active filters over passive filters. |  |
| e) | Draw the circuit of a Monostable multivibrator using IC 555. |  |
| f) | List the features of the 78xx series fixed voltage regulators. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain with a circuit diagram a dual input unbalanced output differential amplifier. |  |
| b) | Draw the circuit of a voltage shunt feedback amplifier. Write the expression for the voltage gain. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the block diagram of an Op-amp. |  |
| b) | Draw the schematic symbol for the Op-amp. Explain the inverting and non-inverting inputs. List 3 characteristics of an ideal Op-amp. |  |
| c) | Define the following Op-amp parameters: i) CMRR ii) Slew rate iii) Bandwidth iv) Input offset voltage v) Input impedance vi) Input bias current. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of an integrator. Draw the output waveform for i) square wave input ii) sine wave input. |  |
| b) | Draw the circuit of Schmitt Trigger and explain its operation. Write expressions for the Upper Threshold voltage and the Lower Threshold voltage. |  |
| c) | Explain the following applications of an Op-amp, with a circuit: i) Zero crossing detector ii) 3 input Summing amplifier. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw the circuit of a First Order Butterworth High Pass Filter. Draw the frequency response for this filter. Write the expression for VOVin. |  |
| b) | Draw the circuit of a First Order Butterworth Low Pass Filter. Write the value of VOVin for i) f < fH ii) f = fH iii) f > fH where fH = cut off frequency of the filter. |  |
| c) | Draw the circuit of a Wein Bridge Oscillator using Op- amp. Write the expression for the frequency of oscillation. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit and explain the working of a square wave generator. Draw the waveforms for the capacitor voltage and the output voltage. |  |
| b) | Draw the circuit and explain the working of an op amp series voltage regulator. |  |
| c) | Explain the working of IC 723 as a high voltage regulator. |  |

***********