## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg./P

Subject: Tool Engineering-I(Cutting Tools)[5033]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why are discontinuous chips preferred over continuous chips? |  |
| b) | What do you mean by Flank wear and Crater wear? |  |
| c) | What do you understand by Tool signature? |  |
| d) | Why relief angles on carbide tools are kept small? |  |
| e) | What are the factors on which selection of cutting fluid depends? |  |
| f) | Explain the meaning of ‘Shear Plane’? |  |
| g) | What are the main factors that influence the tool life? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive an expression to show the relationship between chip thickness ratio and shear angle. |  |
| b) | Discuss about the influence of different variables on machineability. |  |
| c) | Explain the following tool signature for a single point cutting tool: 10 – 20 – 7 – 6 – 8 –15 - 0.8mm. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Show the various elements of the following with neat sketches: i) Taps ii) Reamers. |  |
| b) | What are the various types of high speed steels used in machinery operations? |  |
| c) | State the functions of cutting fluids. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | While machining on a shaper under orthogonal cutting conditions the specimen length along the stroke is 100mm and the chip length of 40mm is obtained with a tool of 15 rake angle. Determine the shear angle and the chip thickness if uncut chip thickness is 1.5mm. |  |
| b) | Explain the concept of Tool failure. |  |
| c) | Explain the properties and applications of cemented carbides. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw Merchants circle diagram for cutting forces on orthogonal cutting. |  |
| b) | Explain optimum cutting speed for maximum production and maximum profit in machinery economics. |  |
| c) | Comment on the storage & disposal of cutting fluids. |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Continuous chip with Built up edge |  |
| b) | Tool geometry |  |
| c) | Turning and Boring tools |  |
| d) | Cutting speed and feed |  |

***********