## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-II(4041)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is supercharging in case of I.C. engines? |  |
| b) | Give classification of Steam Turbines. |  |
| c) | Define: Psychrometry, COP, 1 Ton of Refrigeration. |  |
| d) | With a neat sketch show Simple Vapour Absorption Refrigeration system. |  |
| e) | Give advantages of Gas Turbine. |  |
| f) | What is Steam Condenser? Give its objectives. |  |
| g) | What is Nozzle efficiency? How is it affected by friction in Nozzle? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain pressurized lubrication system in IC engine. |  |
| b) | An air-Standard diesel cycle has compression ratio of 14. The pressure at the beginning of compression stroke is 1 bar and the temperature is 27C. Maximum temperature of cycle is 2500C. Determine the efficiency of the engine. |  |
| c) | Compare 2 stroke engine with 4 stroke engine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of a 4 stroke petrol engine. |  |
| b) | Write a note on cooling of IC engines. |  |
| c) | Following observations are made during trial on 4 stroke, 4 cylinder, petrol engine: Diameter of cylinder = 102mm Stroke =114mm Speed = 1500 rpm Fuel consumption = 0.200Kg/min. Calorific value = 42000KJ/Kg Net Brake Load = 380N Radius of Brake Drum = 0.5m Mechanical Efficiency = 83%. Find i) Brake Thermal Efficiency ii) Indicated thermal Efficiency iii) Brake specific fuel consumption. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Steam issues from nozzle of a single stage impulse turbine at 1500 m/s and nozzles are inclined at 25 to direction of motion of blades, which have a speed of 400 m/s. The blade at inlet and outlet are equal. If flow over blades is frictionless, find inlet blade angle, force exerted on blades in direction of motion and power developed when steam flows at the rate of 4000 Kg/hr. |  |
| b) | Explain Pressure Compounding in case of impulse turbine with the help of Pressure-Velocity diagram. |  |
| c) | Give the advantages of Steam turbine. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by super saturated flow in Steam Nozzle? What are the effects of super saturation? |  |
| b) | With a neat sketch explain working of low level jet condenser. |  |
| c) | Explain construction and working of any cooling Tower with a sketch. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare Gas Turbines with Steam Turbines. |  |
| b) | What are Refrigerants? Give their desirable properties. |  |
| c) | Write a short note on cooling load estimation and sources of heat load. |  |

***********