## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./COMP/EC/EI

Subject: Basic Electrical Engineering(CS304) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw sinusoidal A.C. voltage waveform and label Time period & maximum voltage. |  |
| b) | Define Transformer. State one application of Transformer. |  |
| c) | Classify D.C. motors and state one application of each. |  |
| d) | State the names of any 3 starters used for 3 phase Squirrel cage Induction Motor. |  |
| e) | State the methods of reducing Earth resistance. |  |
| f) | Define Fuse and state the different types of Fuses. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain Real Power, Reactive Power and Apparent Power in three-phase system. |  |
| b) | With a neat diagram, explain the basic construction of single phase transformer. |  |
| c) | What are the various losses in Transformer? Which are the constant losses and how they are minimized. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With neat diagrams, explain reversal of direction of rotation of D.C. shunt motor. State one application of D.C. shunt motor. |  |
| b) | Draw and explain Torque v/s Armature current characteristics of D.C. series motor. |  |
| c) | Draw and explain Speed v/s Torque characteristics of D.C. series motor. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With a neat circuit diagram, explain the working of Direct-on-Line starter used for 3 phase Induction motor. |  |
| b) | Why starter is necessary for 3 phase Induction motor? |  |
| c) | Explain principle of operation of 3 phase Induction motor. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Why single phase Induction motor is not self–starting? What are the methods used to make the single phase motors self-starting? |  |
| b) | Write a note on ‘Universal Motor’. |  |
| c) | Write neat diagram, explain ‘Pipe Earthing’. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the principle of operation of E.L.C.B. (Earth Leakage Circuit Breaker) |  |
| b) | Write a note on ‘Fuse’. |  |
| c) | Write a note on M.C.B. |  |

***********