## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./EC/EI/MED

Subject: Basic Electronics Engineering(CS308) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is ripple with respect to output waveform of rectifier? Define Ripple factor. |  |
| b) | Explain why an ordinary junction transistor is called Bipolar Junction Transistor. |  |
| c) | Draw the symbol and structure of enhancement and depletion type of MOSFET |  |
| d) | Define propagation delay, one of the characteristics of Digital IC Logic family. |  |
| e) | Explain why Zener diode is used as a voltage regulator. |  |
| f) | What causes collector current to flow when the emitter current is zero? What is this collector current called? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of capacitor filter for a rectifier with neat waveforms. Draw necessary diagrams. |  |
| b) | Explain the working of Zener diode voltage regulator circuit under following conditions: i) Fixed applied voltage & variable load ii) Fixed load and variable applied voltage. |  |
| c) | A Zener voltage regulator circuit is drawing current from a variable voltage supply of (80-120V). Zener diode has Vz = 50V and if series resistance is 5 K and load resistance is 10 K, find maximum and minimum values of Zener diode current. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the output characteristics of a transistor in CB configuration and explain. |  |
| b) | Compare the three transistor configurations. |  |
| c) | Explain the basic functioning of an NPN transistor and show that IE = IB + IC. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and Explain output characteristics of N-channel JFET. |  |
| b) | Draw and Explain constructional details of n-channel E-MOSFET also explain its operation in the circuit when VGS = 0V and VGS > 0. |  |
| c) | Draw and explain D-MOSFET transfer characteristic. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat diagram of TTL NAND gate and explain its circuit operation in brief. |  |
| b) | Explain following characteristics of digital logic IC: i) Fan in ii) Fan out iii) Power dissipation iv) Noise Immunity. |  |
| c) | Draw neat diagram of CMOS NOR gate and explain working operation in brief. | ……2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Zener diode VI characteristics |  |
| b) | N-type semiconductor |  |
| c) | Expression of Drain current in JFET |  |
| d) | Static and Dynamic memory |  |
| e) | Advantages of FET over BJT |  |

***********