## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg./A/SB

Subject: Strength of Materials(4032) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A M.S tie bar 75mm in depth 15mm thick and 1 metre long is subjected to an axial force of 250 KN. The elongation of tie bar is found to be 0.758mm. Find the modulus of elasticity for the material? |  |
| b) | What is the value of modulus of rigidity for the material whose modulus of elasticity is 180GPa and Poisson’s ratio is 0.25? |  |
| c) | Explain the principle of complimentary shear stresses? |  |
| d) | Show that the maximum intensity of stress developed due to suddenly applied load is twice the stress developed due to gradually applied load on the material. |  |
| e) | Define: (i) Neutral axis (ii) Moment of Resistance (iii) Modulus of section. |  |
| f) | Explain the term ‘Buckling’ in columns? |  |
| g) | What are the assumptions made in the ‘Theory of pure Torsion’? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A member ABCD is subjected to point loads P1, P2, P3 and P4 as shown in fig.1. Calculate the force P2 necessary for equilibrium if P1=450KN, P3=130KN and P4=45KN. Also determine the total elongation of the member if E=210 KN/mm2. |  |
| b) | A tube of Aluminium having 50mm OD & 25mm ID is snugly fitted to the solid rod 25mm in diameter. The composite bar is loaded in compression by load P. Find stress developed in Aluminium tube if stress developed in solid steel rod is 80 MPa. Also find magnitude of applied load P. Take E steel=2 x105 MPa, EAL=7 x104 MPa. |  |
|  |  | ….2/- |
|  | -2- |  |
| c) | In construction of a railway track a steel rail of 12m long is laid at a temperature of 25C. It is expected that maximum rise in temperature is 55C of this track. - Find the minimum gap to be kept between two rails so that thermal stresses does not develop. - Calculate the thermal stresses developed in the rails if: 1) No expansion gap is provided between the rails. 2) If a gap of 2mm is provided for expansion. =10x10-6 /C |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The principal stresses at a point in a steel bar are 250MPa (tensile) and 125MPa (compressive). Determine the resultant stress in magnitude & direction on a plane inclined at 50 to the axis of major principal stress. Also determine magnitude of maximum shear stress. Solve using Mohr’s circle method. |  |
| b) | A point in a machine part is subjected to a tensile stress of 65N/mm2 and a compressive stress of 5000N/cm2 acting on 2 mutually perpendicular planes and complimentary shear stress of 15x106 N/m2. Determine the principal stresses and maximum shear stress? |  |
| c) | A bar of 2.5m length having 1600mm2 cross-sectional area hangs vertically and has a collar fixed at its lower end. Find the stress induced in the bar when a weight of 5KN falls from a height of 200mm on the collar. Also find the strain energy stored in the bar. Take E=120 GPa. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the shear force and bending moment diagram for the cantilever beam shown in fig.2. |  |
| b) | A beam AB is a simply supported beam having a span of 10m. It carries a point load of 6KN at a distance of 3m from A and another point load of 8KN at 7m from A. Also UDL of 1KN/m acts on beam between these point loads. Draw the shear force, bending moment & load diagrams for this beam. |  |
| c) | Determine the moment of Inertia about the centroidal Y-Y axis of the angle section as shown in fig.3. Also find the moment of inertia about the left most fibre. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Moment of Inertia of I section of a beam 500mm depth is 69.49 x 107mm4 about its centroidal axis. Find the longest span over which a beam of this section when simply supported, could carry a UDL of 50KN/m. The flange stress in the material should not exceed 110 MPa. |  |
| b) | A bar of rectangular cross-section 100mm x 50mm carries an axial compressive load of 180KN. If the line of action of load is 12.5mm from the centre of thickness of the section along the centre line of the breadth, find maximum & minimum stresses in the bar. | ……3/- |

|  | -3- |  |
| --- | --- | --- |
| c) | A solid steel shaft transmits 25KW of power at 1200r.p.m. What should be the minimum diameter of this shaft if the permissible shear stress for the shaft material is 60MPa. If this solid shaft is replaced by hollow shaft having outer diameter 1.25 times inner diameter of shaft, what will be its inner & outer diameter? Also find percentage reduction in weight of hollow shaft as compared to solid shaft/metre length. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A thin cylindrical shell of thickness 6mm & diameter 400mm is subjected to an internal pressure which produces a strain 1/2500 in the diameter. Find this internal pressure? Also find the consequent hoop & longitudinal stress. Take E=2x105 MPa, µ=0.3. |  |
| b) | Find Euler’s critical load for a hollow cylindrical C.I column 225mm external diameter and 25mm thick, if it is 6m long, fixed at one end & hinged at the other end. Take E=8x104MPa. Compare Euler’s critical load with Rankine’s critical load if c=550MPa, =1/1600. |  |
| c) | A simply supported beam AB of span 5m carries a point load of 4KN of a distance of 2m from the end A. Determine the slopes at A & B. Also determine deflection under the load. Take EI=30x1012 N-mm2. |  |