## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Microprocessor Based Instrumentation(5114)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the term ‘Instruction queue’ used in 8086 architecture and state its advantages. |  |
| b) | Explain salient features of a microcontroller. How it is different from microprocessor? |  |
| c) | Enlist some applications of microcontrollers. |  |
| d) | What are the addresses of accumulator and PSW in RAM of 8051 microcontroller? Explain functionality of each. |  |
| e) | Mention at least 2 jump instructions of 8051. Compare them in brief. |  |
| f) | When does ‘overflow’ flag sets in 8051 microcontroller? Explain with example. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat functional block diagram of 8086 processor and explain. |  |
| b) | What is memory segmentation? Explain its advantages with reference to the 8086 processor. |  |
| c) | Explain the different addressing modes of 8086 processor and give example of each. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat diagram and explain how external memory is connected to 8051 microcontroller and how it is accessed. |  |
| b) | Draw the internal block diagram of 8051 and list out all the features of this microcontroller. |  |
| c) | Explain the input-output port pins and their functions, with the help of a neat circuit diagram of a port pin. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the importance of INT0and INT1 interrupts of 8051 microcontroller. How 8051 can be interrupted by external device at INT0 pin? |  |
| b) | Explain the modes of operations used for serial data transmission in 8051 microcontroller. |  |
| c) | With the help of formats explain each bit of TCON and TMOD registers. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following instructions with an example: i) MOV DPTR, # nn ii) MOV @ Rp, # n iii) XCHD A, @ Rp iv) ADDC A, add v) RLC A vi) CJNE A, # n, radd |  |
| b) | Write a short note on Logic and compare instructions of 8051 microcontrollers. |  |
|  |  | ……..2/- |
|  | -2- |  |
| c) | Write the content of stack pointer and top of stack, after execution of each instruction given below. MVI A, #50 MVI SP, #30 PUSH A PUSH SP |  |
|  | POP A |  |
|  | POP SP |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write assembly language program to transfer data from external memory location starting from 2000H to 2010H to internal RAM location starting at 104 (decimal number). |  |
| b) | Write a program to count the number of ‘one’ s in a byte stored at RAM location of 8051 microcontroller at address 04H and store the result in Ro of 8051. |  |
| c) | Write instructions to add the bytes present in R1 and R2 and store the same in R3 and carry bit in R4. |  |
| d) | Data bytes are stored in RAM location of 8051 microcontroller from 10H to 18H. Write a program to search data for FFHand save 00H at that memory location. |  |

***********