## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT

Subject: Concrete Technology(4007) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the appropriate use of following types of concrete in construction: i) Cement concrete ii) RCC ii) Pre-stressed concrete. |  |
| b) | What precautions you will take during storage of cement to prevent possible damage? |  |
| c) | Explain the following terms: i) Fineness of cement ii) Hydration iii) Setting and hardening. |  |
| d) | When and where ‘Construction joints’ and ‘Expansion joints’ are used in a structure? |  |
| e) | What is meant by “Ready Mix Concrete”? When and where it is used? |  |
| f) | State the minimum time of removal of form work after concreting in case of following: i) RCC Slab ii) RCC Beam iii) RCC column. |  |
| g) | Explain the purpose of conducting ‘Non-destructive test’. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | How the cement is manufactured in a cement plant? Give an outline diagram showing the manufacturing process of Portland cement. |  |
| b) | Give a list of tests conducted for cement in the laboratory with their acceptability of test results for each. |  |
| c) | What is ‘fineness modulus’? How will you conduct ‘Sieve analysis’ test for coarse aggregates? |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the following operations in concrete making: i) Proportioning of ingredients ii) Types of mixing concrete. |  |
| b) | State the object of ‘compaction of concrete’. How it is done at site manually and by mechanical means? |  |
| c) | What is meant by ‘Workability’ of concrete? Explain how it is checked at the field. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is meant by an ‘admixture’? Name the different types of admixtures with their specific use. |  |
| b) | Explain with neat sketches, the different types of joints used in a building. Also state the merits of use of such joints. |  |
| c) | State the properties and advantages of following types of special concrete: i) Pre-cast concrete ii) Light weight concrete iii) Fibre reinforced concrete. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Mention the properties and use of following: i) Air-entrained concrete ii) Ferrocement concrete. |  |
| b) | What are the basic requirements of form work? Also mention the types of form work used for multistoreyed structures. |  |
| c) | State the objectives of concrete mix design. List the methods and explain any one in detail. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Re-bound hammer test |  |
| b) | Determination of coarse and fine aggregate ratio |  |
| c) | Different grades of concrete and their specific use |  |
| d) | Determination of water cement ratio |  |
| e) | Precautions during removal of form work |  |

***********