## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg,/CT

Subject: Construction-I(4004) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat sketch of a dry stone masonry wall. Where are these walls used? |  |
| b) | Define ‘Bond in Brick Masonry’. Why should bond be provided? |  |
| c) | Explain the terms hip, eaves and gable. |  |
| d) | Write a note on PVC doors. Where are these doors used? |  |
| e) | Draw a neat sketch of a stair and name the following: (i) Nosing (ii) Riser (iii) Landing (iv) Balusters (v) Soffit (vi) Handrail. |  |
| f) | What is a mezzanine floor? Where is it located? |  |
| g) | Explain the term ‘neeru finish’. Why is it done? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State six characteristics that a good building stone should possess. |  |
| b) | Draw a neat sketch of a wall footing. Also explain all the different types of loads coming on the footing. |  |
| c) | Define ‘Bearing capacity of soil’. State the values of ‘Bearing capacity’ in different types of soils. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of an English Bond. What are the points to be remembered while constructing the English bond? |  |
| b) | Draw neat sketch of i) Openings in brickwork spanned by arches ii) Lean-to-roof. |  |
| c) | Name the different types of covering used for sloping roofs. Explain the procedure for laying GI sheets. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch Top Hung, Bottom Hung and Middle pivoted ventilators. |  |
| b) | Write notes on: i) Flush door ii) IPS flooring. |  |
| c) | Explain ‘Jack Arch Floor’ with a neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the requirements of a good stair. |  |
| b) | Write a note on preparation of surfaces for plastering. |  |
| c) | What is pointing? Why is it done? |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Instruments for excavation |  |
| b) | Setting out methods |  |
| c) | Grades of concrete for different uses |  |
| d) | Advantages of concrete over other materials |  |

***********