## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Mechanical Engg./A

Subject: Thermal Engineering(ME303) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric charts, Steam tables and Mollier’s charts is

permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between extensive and intensive properties. Give example of each and state their units. |  |
| b) | State different type of thermodynamic systems. Give example of each type. |  |
| c) | State and explain Clausius statement of second law of thermodynamic. |  |
| d) | State characteristic equation of a gas. State units of all the quantities involved. |  |
| e) | Name any three boiler mountings and state the function of each mounting. |  |
| f) | How are steam turbines classified? |  |
| g) | Give the classification of steam condensers. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cold storage is to be maintained at -5C while the surroundings are at 35C. The heat leakage from the surroundings into the cold storage is estimated to be 29KW. The actual C.O.P. of the refrigeration plant is one-third of an ideal plant working between the same temperatures. Find the power required to drive the plant. |  |
| b) | A certain gas occupies O.15m3 at a temperature of 20C and a pressure of 1.2 bar. If the gas has a mass of 0.2 Kg, calculate the value of gas constant and molecular mass of the gas. Take universal gas constant = 8.314 KJ/KgK. |  |
| c) | With the help of a neat sketch, explain the construction and working of a throttling calorimeter. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | 0.25m3 of gas at 288K and 100KPa is compressed adiabatically to 700 KPa. Calculate i) The final temperature of the gas ii) The work done on the gas. Take Cp = 1.001 KJ/KgK and Cv = 0.715 KJ/KgK for the gas. |  |
| b) | 0.4 Kg of air at a pressure of 100KPa and a temperature of 288K is compressed isothermally to 500 KPa. Calculate the final volume and the heat rejected during the compression process. Take R= 0.287 KJ/KgK for air. |  |
| c) | With the help of neat sketch, explain the construction and working of a Cochran boiler. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate the internal energy of 1Kg of steam at a pressure of 8 bar when i) The steam is 0.8 dry ii) The steam is superheated and the temperature of steam is 270C. Take specific heat of superheated steam at constant pressure as 2.2 KJ/KgK. | ….2/- |
|  | -2- |  |
| b) | A steam power plant is supplied with dry saturated steam at a pressure of 14 bar and exhausts into a condenser at a pressure of 0.3 bar. Using steam tables, calculate the efficiency of Rankine Cycle for these conditions. Draw T-S diagram for the cycle. |  |
| c) | With the help of a neat schematic, explain the construction and working of La-Mont boiler. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketches, explain the construction and working of i) Fusible plug ii) Feed check valve. |  |
| b) | Explain the concepts of reheating and regenerative feed heating in steam turbines. |  |
| c) | With the help of a neat sketch, explain the construction and working of a forced draft cooling tower. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain i) Kinetic energy ii) Internal energy. |  |
| b) | What is a heat pump? Derive an expression for its C.O.P. |  |
| c) | A certain quantity of air is cooled at a constant pressure from 300K to 280K. If the initial volume of the air is 0.15m3, find by how much the volume will diminish. |  |
| d) | State different types of boiler draught. Explain any one type. |  |
| e) | Explain the meaning of compounding of turbines. |  |
| f) | Define i) Condenser efficiency ii) Vacuum efficiency. |  |

***********