## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical & Electronics Engg.

Subject: Power Generation, Transmission & Distribution(5091)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why is a moderator necessary in a nuclear power plant? |  |
| b) | Define (i) Load factor (ii) Utilization factor. |  |
| c) | Why bundled conductors are used for overhead transmission? |  |
| d) | State any three drawbacks of EHV transmission system. |  |
| e) | Define the terms feeder, distributor and service mains. |  |
| f) | State the factors affecting earth resistance. |  |
| g) | What is cleat wiring? State its applications. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A proposed power station has following load cycle Time (hrs) 0-6 6-10 10-12 12-16 16-20 20-24 Load (MW) 40 50 60 50 70 40 Draw the load curve and find (i) Units generated per day (ii) average load and (iii) load factor. |  |
| b) | With a neat diagram explain working of hydel power plant. |  |
| c) | State the advantages and disadvantages of diesel power plant. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is corona? State advantages and disadvantages of corona. |  |
| b) | A 3 phase overhead transmission line is being supported by three disc insulators. Potential across top unit i.e near to tower and middle unit are 8KV and 11KV respectively. Calculate (i) ratio of capacitance between pin and earth to mutual capacitance of each unit (ii) line voltage (iii) string efficiency. |  |
| c) | Explain in brief different types of supports used in transmission systems. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Compare radial, ring and interconnected system on the basis of load, reliability of supply and economy. |  |
| b) | Explain the classification of distribution system with respect to voltage and number of wire. Briefly explain the DC 2 wire system of distribution. |  |
| c) | Write a short note on system voltage and permissible voltage drop in various parts of an urban and semi urban distribution system. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Compare PVC casing capping wiring system with conduit wiring system. |  |
| b) | Discuss the IE rules for domestic wiring system. |  |
| c) | With a neat diagram explain pipe earthing. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | State various methods of improving string efficiency. |  |
| b) | Write a short note on the following: - ACSR conductors - Double earthing. |  |
| c) | With a neat diagram explain: - Stair case light circuit - Godown wiring. |  |

***********