## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Common

Subject: Elements of Civil Engineering(3003)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Ranging. Explain in brief, what is meant by reciprocal ranging and where we get the situation. |  |
| b) | If a distance of 300.00 m is measured by a faulty chain that is too long by 20 cms, what will be the measured faulty length? |  |
| c) | Explain the method to measure a distance between two points across a building (as obstruction). |  |
| d) | What are the two systems of measurement of bearings? How do you differentiate between a bearing and an angle? |  |
| e) | Convert the following bearings into the other system. 190 40, 135 20, 25 15, 300 25. |  |
| f) | Mention any three types of roof trusses with single line sketches. |  |
| g) | Draw the section of a simple spread footing for a wall. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enlist the parts of a dumpy level. Specify the smallest division on a leveling staff. |  |
| b) | In a leveling work, the following readings are taken by a dumpy level at every 30 m chainage on a continuous sloping ground. 1.785, 1.990, 2.255, 2.805, 3.560, 1.550, 2.155, 2.970, 3.400, 2.390, 3.110. |  |
|  | The first reading was taken on zero chainage & on a B.M. of 100.00m. |  |
|  | Prepare a standard levelling sheet, enter all the readings properly and calculate the R.L.S of all the points. |  |
| c) | Differentiate between: |  |
|  | i) A level line and a horizontal line at a point. |  |
|  | ii) Plunging and swinging of a theodolite. |  |
|  | iii) Normal position and inverted position of a theodolite. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain stepwise and with sketch, the method of measurement of a horizontal angle by method of repitition, using a theodolite. |  |
| b) | Define Safe Bearing Capacity (S.B.C) of soil. Specify some common values of S.B.C. for moderately hard soil. Write a formula for depth of foundation in relation with angle of repose () of soil. |  |
| c) | Mention any three types of foundation and situations where each is suitable. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Name any five types of flooring. Explain method of construction of I.P.S flooring (P.C.C. flooring). |  |

………2/-

-2-

| b) | Write any six points to be followed while concreting to produce good quality concrete structures. |  |
| --- | --- | --- |
| c) | What is meant by R.C.C. 1:3:6, 1:2:4, 1:11/2:3? Specify mortar mixes for plastering and for construction of masonry work. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by sound insulation and thermal insulation in buildings? Specify some materials used in building for thermal insulation. |  |
| b) | Explain some methods of providing sound insulation and acoustic methods in buildings. |  |
| c) | Draw a neat plan and section of a R.C.C Raft foundation showing reinforcements. Also mention where the raft foundations are suitable? |  |
| Q.No.6. | Develop a neat plan of a building referring to the line plan shown in fig.1, to a scale of 1:50. Take the wall thickness as 20 cms. Show the dotted chajjas, kitchen platform and sink. Write the details of each room with dimensions internally and show the external dimensions. | (12) |
|  |  | ….3/- |