## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engg./E/EI/EC/COMP/MED/E&EE

Subject: Basic Electronics(4132)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain with neat diagram electron energies in an atom. |  |
| b) | What do you mean by extrinsic semiconductor? Mention two examples. |  |
| c) | Define ripple factor and rectification efficiency of rectifier. |  |
| d) | Define alpha & beta of BJT. |  |
| e) | State three advantages of CE configuration of transistor. |  |
| f) | Draw a symbol of n-channel JFET & depletion type MOSFET. |  |
| g) | Define triode parameters , rp & gm. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain zener diode as a regular. |  |
| b) | Calculate PIV rating of diodes used in Full wave rectifier shown in fig.(1). Find maximum dc voltage that can be obtained from this circuit. Determine dc current through the load. |  |
| c) | Explain with neat diagram the working of bridge rectifier. Draw its o/p waveforms. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the formation of N type semiconductor. |  |
| b) | Why filter circuits are required? Explain ‘L’ filter with neat o/p waveforms. |  |
| c) | Explain forward biasing of diode. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the comparison of three transistor configurations. At least 4 points should be explained. |  |
| b) | Why do we bias the transistor? Explain fixed bias circuit in detail. Why it is rarely used? |  |
| c) | Which are the three different regions of transistor? What is DC load line? How to plot it? |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain, why Q point of transistor is fixed at the centre of active region of o/p characteristics of transistor. |  |
| b) i) | In a certain transistor dc = 0.98, IE = 2mA. Calculate IC & IB. |  |
| ii) | If IE = 2mA & leakage current ICBO =5A, dc = 0.985. Calculate IC & IB. |  |
| c) i) | What do you understand by the term ‘channel’ in JFET? |  |
| ii) | Define all the important parameters of JFET. |  |
| d) | Explain working of NPN transistor, with neat diagram. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Write a note on Vacuum triode. |  |
| b) | Explain the term ‘thermionic Emission’. Name other types of emission possible. |  |
| c) | Explain Enhancement MOSFET with its characteristics curves. |  |

……2/-