## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Automobile Engineering

Subject: Automobile Engines-I(4054)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Compression Ratio. If the swept volume of a single cylinder SI engine is 98cc and the clearance volume is 12cc. Calculate the Compression Ratio. |  |
| b) | Define Cetane Number. How is it determined for an unknown fuel? |  |
| c) | Classify fuels for IC. Engines and give the merits and drawbacks of each. |  |
| d) | Explain at least three types of fans for I.C. engine water cooling. |  |
| e) | Compare 2-stroke and 4-strokes I.C. engines w.r.t. thermal efficiency, lubrication, power and size of flywheel. |  |
| f) | List the qualities desired of a fuel for the C.I. engine, briefly explaining each one. |  |
| g) | Distinguish piston slap and engine knock. |  |
| h) | List the factors on which heat dissipated in any engine cooling, depends. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | With neat sketches explain the working principle of a 2–stroke S.I. Engine. |  |
| b) | Explain the construction and functions of flywheel and vibration damper. |  |
| c) | Explain with sketches timing gears, timing chain and timing belt. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | With a neat labelled sketch explain the construction and working of an Elementary Carburetor. |  |
| b) | Explain the MPFI system, listing its advantages and disadvantages. |  |
| c) | Describe clearly the Air-Petrol mixture ratios required for the steady state operation of a S. I. Engine. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Describe any two methods of cooling of an I.C. engine supported by neat sketches. |  |
| b) | Explain with sketches the air injection system and one type of Airless injection for Diesel Engines. |  |
| c) | What is an injector of a CI Engine? Explain its construction and working with the help of a neat diagram. |  |

………2/-

-2-

| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw a neat diagram of a Dry Sump Lubrication and explain its working. |  |
| b) | Describe the different types of S.I. Engine combustion chambers with the help of sketches. |  |
| c) | Plotting a neat p-t graph, explain the CI Engine ignition delay and phases of combustion. |  |
| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| a) | What are the functions of Lubrication? List all types of lubricants. |  |
| b) | What is brake power? Describe the calculation of Brake power using a neatly drawn Rope Brake Dynamometer. |  |
| c) i) | Explain the Swinging field d.c. Dynamometer. |  |
| ii) | A 4-cylinder 4-stroke Petrol Engine having Bore x stroke as 100 mm x 98 mm and Compression Ratio as 8.5:1, has a Volumetric Efficiency of 90%. Calculate the total swept volume per cycle and clearance volume of each cylinder. |  |

***********