## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Theory of Structures-II(4023)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | A fixed beam of span ‘l’ carries an udl throughout. Sketch BMD and SFD of the beam. |  |
| b) | Define effective length and state the values of effective length for various end conditions of column. |  |
| c) | State the limitations of Euler’s formula. |  |
| d) | Define Relative stiffness with examples. |  |
| e) | A three hinged arch of 8 m span with central rise 1.5m carry udl throughout of 25KN/m. Find reaction at supports. |  |
| f) | Explain the moment distribution method of analysis of continuous beams. |  |
| Q.No.2. | Answer any one of the following: | 1 x 12 = 12 |
| a) | A cantilever beam of 4m span carries an udl of 20KN/m throughout the span along with point load of 16KN at centre of beam. Beam section has E=200KN/mm2 and Ixx=3442 x 104 mm4. Find maximum slope and deflection. |  |
| b) | A simply supported beam of 6m length carries an UDL of 30KN/m throughout along with point load of 20 KN at 2m from left end. Find maximum deflection in beam. Also slope at left end. Take E=0.15x105 N/mm2 and size of beam 200 x 250m. |  |
| Q.No.3. | Answer any one of the following: | 1 x 12 = 12 |
| a) | A fixed beam of length 8 m carries point loads of 40KN, 60KN and 10KN at a distances of 2m, 4m and 6m from left end. Find Fixed end moments and draw SFD and BMD locating point of contraflexure. |  |
| b) | A fixed beam of 6m span carries an UDL of 40KN/m for a length of 3 m placed symmetrically in the middle of beam. Find Fixed End moments and sketch SFD and BMD. |  |
| Q.No.4. | Answer any one of the following: | 1 x 12 = 12 |
| a) | Analyse a continuous beam ABCD with all supports A, B, C and D as simple supports. The length of spans AB=BC=CD=5m. There is an UDL of 30 KN/m throughout ABCD portion. EI is uniform for all beams. Draw SFD and BMD. |  |
| b) | A continuous beam ABC has end A fixed and B as simple support and BC as overhang. AB=6m and BC =2.5m. There is UDL of 35KN/m throughout ABC length along with point load of 10KN at free end C. Take EI same for all. Draw SFD and BMD. Use moment distribution method analysis. |  |

……2/-

-2-

| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | An I section has flanges 200 x 12.5mm and web 225 x 10 mm. The height of column is 6m with one end fixed, other hinged. Find safe load using Rankine’s formula. Take fc =320N/mm2 and Rankine’s constant = 1/7500 and F.O.S. = 2.5. |  |
| b) | A hollow circular column with internal diameter 0.75 times outer diameter carries axial load of 250KN. Using factor of safety as 5, find size of column. Effective height of column is 5m. Use Euler’s formula. Take E=1x105 N/mm2. |  |
| c) | Find the maximum length of steel rod of 50 mm diameter, used as column with both end fixed and carrying of a load of 28 KN. Allow factor of safety 3. Take =1/7000 and fc = 300N/mm2. |  |
| Q.No.6. | Answer any one of the following: | 1 x 12 = 12 |
| a) | A three hinged arch of span 10 m and central rise of 2m with both supports at same level carry point loads of 40KN, 60KN and 20KN at a distances of 2m, 4m and 6m from left support. Find (i) reaction at supports ii) Bending moment under all the point loads. |  |
| b) | A three hinged arch of span 16m with rise 4m carries an UDL of 65 KN/m for the left half span. Also a point load of 50 KN at 4m from right support. Find (i) reaction at supports |  |
|  | (ii)Normal thrust and radial shear at 4m from left end of arch. |  |

***********