## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Estimation & Costing(4242)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the concept of Estimating. |  |
| b) | Define the following terms used in machining: i) Cutting speed ii) Feed iii) Depth of cut. |  |
| c) | Explain the factors affecting welding cost. |  |
| d) | Explain in short different sheet metal joints. |  |
| e) | Explain the reasons for replacement of equipments. |  |
| f) | Define i) Informal Tender ii) Unbalanced tender. |  |
| g) | Define a contract and state the essentials of contract. |  |
| h) | State the conditions where whole of Security Deposit is forfeited. |  |
| Q.No.2. | Prepare a measurement page and an abstract page for a “Roof Truss” used in an industrial shed for the following items. Refer Fig.1. | (12) |
|  | 1.Roof sheet coverings. |  |
|  | 2.Structural steel of rolled steel sections |  |
|  | 3.Structural steel of plates. Size of Industrial Building 4 x 24m. Spacing of Truss 3m c/c. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Calculate the time required to machine the job shown in Fig. 3 of mild steel from a 60mm diameter round stock of length 105mm. Assume suitable machining parameters. | (08) |
| b) | What is Break-even theory? | (04) |
|  | ### OR |  |
| b) | Explain the Estimating procedure. | (04) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain different sheet metal operations. |  |

………2/-

-2-

| b) | A mechanical press was purchased by a company for Rs. 22,00,000/- with an expected useful life of 10,000 hours with a scrap value of Rs.2,00,000/-. It performs maximum 4 strokes per minute. Calculate the cost of a single stroke if an electric motor of 5 KWH drives the press & the energy charges is Rs. 10/unit. |  |
| --- | --- | --- |
| c) | Calculate the energy charges of a diesel engine driven welding generator having an engine of 5BHP. The output arc current was 105 Amps and arc voltage of 22 volts when used for 3 hours. The engine consumes 50ml of diesel per BHP and the cost of the diesel is Rs. 45 per litre. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A machinery was purchased by a company for Rs.50,000/- with an expected useful life of 5 years. Calculate the annual depreciation by sinking fund method if the rate of interest is 8% and salvage value is Rs. 7000/-. Also calculate the depreciation amount after 3 years. |  |
| b) | Differentiate between Estimation and costing of a product. Explain the causes of depreciation. |  |
| c) | Estimate the cost of material of the job shown in Fig.3 made up of mild steel costing Rs.200/Kg and having specific weight of 7.85gms/cc. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draft a Tender Notice for fabrication work of a water tank structure including a erection for an approximate cost of Rs.12 lakhs. |  |
| b) | Explain briefly i) Earnest money ii) Breach of contract. |  |
| c) | Prepare a measurement page for Slab Base plate connection used for a steel column of 10m height for following items. i) Structural steel ii) anchor bolts. Refer Fig.2. |  |
|  |  | ……3/- |