## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg.

Subject: Power Plant Engineering(5037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the plant ‘Use Factor’ and plant ‘Capacity Factor’ and state their uses in the design and operation of the power plant. |  |
| b) | How the hydro-electric power plants are classified? |  |
| c) | Why the maximum temperature in the gas turbine cycle is limited to 850º C? |  |
| d) | What are the merits of Cyclone Burner over the other methods of burning coal? |  |
| e) | Why the feed water treatment is necessary before supplying to the boiler? |  |
| f) | What is the mechanism of evaporative cooling? |  |
| g) | What are the deirable properties of a good moderator used in Nuclear reactors? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The daily load of a power plant is given below: Time; 12 night; 2 A.M.; 6 A.M.; 8 A.M.; 12 noon; 12.30 P.M.; 1 P.M.; 5 P.M.; 6 P.M.; 12 night. Load (MW); 20; 10; 10; 50; 50; 40; 50; 50; 70; 20. |
|  | Plot load curve and find the load factor. |  |
| b) | What are the functions of following in a storage type hydro- electric power plant? i) Dam ii) Surge tank iii) Draft tube. |  |
| c) | Draw a layout of low capacity diesel power plant. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the working of a simple constant pressure open-cycle gas turbine plant giving a neat sketch. |  |
| b) | Classify the Draught system used in boilers. Explain any one with a neat sketch. |  |
| c) | With a neat sketch explain the construction and principle of working of ‘Electrostatic precipitator’. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between Unit system and Bin system of pulverized coal handling. |  |
| b) | Draw Layout of modern coal based steam power plant. |  |
| c) | With a neat sketch explain Demineralisation process used for water treatment. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the working principle of Induced-draught cross flow cooling tower. |  |

………2/-

-2-

| b) | Pressurized Water Reactor is a self-regulating reactor and can be used for variable loads whereas Boiling Water Reactor is only useful for base load plants. Justify the statement giving reasons. |  |
| --- | --- | --- |
| c) | i) What are the advantages of liquid fuel over solid fuel? |  |
|  | ii)Differentiate between cross-flow and count flow type cooling towers. |  |
| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| a) | Selection of the number and size of unit |  |
| b) | Geological investigation with reference to site selection criteria for a Hydel plant |  |
| c) | Lube-Oil system of I.C. Engine |  |
| d) | Water tests and analysis |  |
| e) | Radio active waste disposal |  |

***********