## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engg./EC/EI

Subject: Electronic Measurements(4137)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Accuracy ii) Precision. |  |
| b) | Explain briefly what is ohms per volt rating. |  |
| c) | Explain why headphones cannot be used as defectors in a DC bridge. |  |
| d) | Briefly explain the use of following controls of CRO: i)Vertical position ii)Trigger level iii)Time/division. |  |
| e) | What is the use of a constant current source in a function generator? |  |
| f) | Briefly compare LED with LCD. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat diagram and explain the working of an unsaturated Weston cell. |  |
| b) | Explain the various damping mechanisms used in a galvanometer. |  |
| c) | A 150v dc source is connected across a series combination of a 100 K and 50 K resistance. Two voltmeters of sensitivity 1 K/V and 20 K/V are used to measure the voltage across the 50 K resistance in the 50V range. Calculate the reading of each meter and the percentage error in both cases. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat circuit diagram and explain the working of a transistor voltmeter. |  |
| b) | Draw a near circuit diagram of a Wheatstone’s bridge and explain its working. Also derive the expression for the unknown resistance. |  |
| c) | Explain with the help of a neat diagram how a Maxwell bridge can be used for finding an unknown inductance. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat block diagram of a two channel CRO and explain its working. |  |
| b) | Explain the following measurements using Lissajous pattern: - Frequency comparison of two signals - Phase differences between two signals. |  |
| c) | Compare in detail the different types of CRO. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat block diagram of a frequency synthesis type of signal generator and explain how it works. |  |
| b) | With the help of a neat block diagram, explain the working of a standard function generator. |  |
| c) | Draw the internal circuit diagram of a common cathode 7 segment LED display and explain how it can be used as a display device. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Classification of standards |  |
| b) | Power supplies in a CRO |  |
| c) | Square wave generator |  |
| d) | Applications of function generator |  |
| e) | Bar graph display |  |

***********