## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Materials & Processes(4238)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the principle of Bessemer’s process of making steel. |  |
| b) | Discuss the effect of silicon and manganese on the properties of steel. |  |
| c) | State the effect of alloying elements on High speed steel. |  |
| d) | How nodular cast iron is produced? State its applications. |  |
| e) | Name the various tests used to identity the metals and alloys. How would you carry out chemical spot testing? |  |
| f) | What is Head-affected zone? Mention its three distinguished regions. |  |
| g) | What is weld decay? What are the methods employed to minimize the problem of weld decay? |  |
| h) | Distinguish between full annealing and process annealing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a solid solution? Discuss the differences between substitutional and interstitial solid solution with a neat sketch. |  |
| b) | Discuss the condition under which Martensite is formed. Why it cannot be shown on iron-carbon equilibrium diagram? |  |
| c) | What are stainless steels? Explain the different kinds of stainless steel with their applications. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the testing method for determining impact properties of metals. |  |
| b) | Explain with the help of a neat sketch the effect of temperature distribution in and around weld. |  |
| c) | Write a short note on corrosion of weld. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the purpose of hardening? Explain the different methods of heating used for hardening process. |  |
| b) | Explain the effect of heating temperature, soaking time and rate of cooling during heat treatment operation |  |
| c) | Explain how properties of aluminum alloys can be improved by Age hardening. State its practical applications. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the effect of strain hardening on the properties of metals. |  |
| b) | Explain the cold pressing and spinning process of metals. |  |
| c) | What do you understand by polymerisation? Differentiate between thermosetting and thermoplastic materials. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any four of the following: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Alloy structural steel |  |
| b) | Malleable cast iron |  |
| c) | Thermo- mechanical stress relief treatment |  |
| d) | Prevention of gaseous contamination of weld |  |
| e) | Normalising |  |
| f) | Insulating materials |  |

***********