## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Common

Subject: Elements of Mechanical Engineering(3004)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is slip in belt drives? |  |
| b) | How are bearings classified? |  |
| c) | How are engines classified? |  |
| d) | State the applications of air motor. |  |
| e) | List the important functions of a lubricant in an engine. |  |
| f) | Define: i) Specific gravity ii) Dynamic viscosity of a fluid. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the formula for length of an i) Open belt drive ii) Cross belt drive. Also derive an expression for Velocity ratio of a simple belt drive. |  |
| b) | A horizontal shaft running at 300 rpm is to drive a parallel shaft at 500 rpm by a belt drive. The pulley on the driving shaft is 60 cm in diameter. If the belt is 6mm thick and 1.5 mm wide, find the diameter of the driven pulley. i) Neglecting slip ii) Assuming a slip of 3% between belt and the pulley. |  |
| c) | With a neat sketch explain the pressure type cooling system of I.C. engine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch and explain with a neat sketch the pressure type system of lubrication used in I.C engines. |  |
| b) | What are the different mountings and accessories used in a boiler? Sketch and explain with a neat sketch any two of them. |  |
| c) | Sketch a Reciprocating air compressor and explain its functioning. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch air cooling system of an I.C. engine. State its advantages and disadvantages. |  |
| b) | State points of difference between water tube and fire tube boilers. Sketch Babbcock and Wilcox boiler and label it. |  |
| c) | How are compressors classified? Explain with a neat sketch multistage compression and why it is done. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | 2 liters of petrol weighs 14N. Calculate its specific weight, mass density, specific volume and specific gravity with respect to water. |  |
| b) | What is priming? Explain with a neat sketch the construction and working of a centrifugal pump. |  |
| c) | Enlist the different types of head losses taking place in a pipe. Show neat sketches wherever required. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Equation for slip in an open belt drive considering ‘t’ (thickness) and formula for power transmitted ‘P’ |  |
| b) | Helical Gears |  |
| c) | Uses of compressed air |  |
| d) | Gauge pressure and vacuum pressure |  |
| e) | Wick type lubrication system |  |

***********