## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Architectural Engg.

Subject: Structures-I(4537)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define force and enlist the characteristics. |  |
| b) | Enlist and explain the three static conditions for equilibrium. |  |
| c) | Define the term couple and explain. |  |
| d) | Enlist the types of loads acting on a structure. |  |
| e) | Explain structural and non-structural cracks. |  |
| f) | Define bending moment and shear force. |  |
| g) | Enlist and explain different grades of concrete. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the bending moment diagram for a cantilever beam of span (l) subjected to a point load at the free end. |  |
| b) | Explain the detailed classification of structure. |  |
| c) | Explain the analytical method of resolution of force system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the magnitude and direction of a single force which will give the same effect on point ‘O’. |  |
|  | 1)10T acting 30 West of south towards ‘O’ |  |
|  | 2)6T due North |  |
|  | 3)8T acting 45º west of North away from ‘O’ |  |
|  | 4)12T acting 60º East of South away from ‘O’. |  |
| b) | Explain the following: |  |
|  | 1)Arches 2) Moment of forces 3) Resultant of force system. |  |
| c) | With the neat sketch, write the CG distance: |  |
|  | 1) Triangle 2) Rectangle 3) Square 4) Circle 5) Semicircle of radius (r). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the B.M.D for a simply supported beam of span 8m, subjected to three points, 20KN, 40KN, 20KN acting at a distance 2m,4m, 6m from left support respectively. |  |
| b) | A cantilever beam AB of span 3m is subjected to two point loads 8KN and 10KN acting at 1.5m and 3m respectively from support(A). Draw the BMD. |  |
| c) | A simply supported beam AB of span 6m is acted upon by a UDL 10KN/m over entire span and 50KN point load at the mid span. Draw B.M.D. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the effect of following on occurrence of crack: - Moisture change - Movement and settlement. |  |
| b) | Explain how the structural cracks can be prevented. |  |
| c) | Explain with example: - Structural and non-structural cracks - Tensile and shear cracks and how these cracks are distinguished. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Sketch the types of beams with different end conditions. |  |
| b) | Draw the following on beam i) point load ii) Uniformly distributed load iii) Uniformly varying load. |  |
| c) | Explain the different grades of steel and its suitability. |  |
| d) | Explain concrete mix design. |  |

***********