## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Airconditioning(5034)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam tables and Psychrometric Charts is permissible.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the term “Tonne of Refrigeration”. Give its value in SI units. |  |
| b) | What are the factors considered for selection of Refrigerants for a system? |  |
| c) | Draw the flow diagram of Basic vapour compression cycle of Refrigeration showing all major components. |  |
| d) | State the advantages of Vapour absorption refrigeration system over Vapour compression refrigeration system. |  |
| e) | Differentiate between - Sensible cooling & sensible heating - Dry bulb & wet bulb temperature. |  |
| f) | Define the following with reference to Air Conditioning: a) Specific humidity b) Absolute humidity c) Enthalpy d) Dew point temperature. |  |
| g) | With the help of neat sketch explain the working of an Air washer. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A R-12 refrigerating machine works on vapour compression cycle. The temperature of refrigerant in the evaporator is -20c. The vapour is dry saturated when it enters the compressor and leaves it in a superheated condition. The condenser temperature is 30C. Assuming cp for R-12 in the superheated condition as 1.884 KJ/KgF, determine - The theoretical C.O.P of the machine. - Compressor work required. The properties of R-12 are as follows: Temperature; Enthalpy; KJ/Kg; Entropy; KJ/KgK. C; liquid; vapour; liquid; vapour. -20; 17.82; 178.73; 0.0731; 0.7087. 30; 64.59; 199.62; 0.2400; 0.6843. |  |
| b) | With the aid of a neat sketch explain the working of a Lithium Bromide Absorption system. |  |
| c) | Discuss the function of a capillary tube in a refrigeration system. Why is it preferred to other throttling devices in household refrigerator? |  |
| Q.No.3. | Answer the following: | 2 x 6 = 12 |
| a) | Explain the working of a thermostatic expansion valve with the help of a neat diagram. |  |
| b) | Describe the working of an evaporative condenser with the help of a neat sketch ### OR |  |
| b) | What do you understand by hermetically sealed compressor? Give the advantages over other refrigerant compressors. | …2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A sample of moist air has a DBT of 25C and a relative humidity of 50%. The Barometric pressure is 740 mm of Hg. Calculate: i)Partial pressure of water vapour and dry air ii)Dew point temperature iii)Specific humidity of air iv)Enthalpy of air per Kg of dry air. |  |
| b) | Show the following air conditioning processes on the skeleton psychrometric chart: - Cooling and dehumidification - Sensible heating - Heating and humidification. |  |
| c) | Define “Bypass factor”. Also explain in brief what do you understand by the terms RSHF and GSHF. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What purpose is served by a fan in an air-conditioning unit? How are fans classified? |  |
| b) | Explain the types of supply air outlets used for air distribution. |  |
| c) | Briefly explain Automatic humidity control, Air movement control, Automatic temperature control, limit switches & time switches. |  |
| Q.No.6. | Answer the following: | 2 x 6 = 12 |
| a) | Draw a simple layout of a cold storage unit or an Ice plant and explain their important components. |  |
| b) | What are the requirements of a good room air distribution? Explain briefly an air handling system with a neat sketch. |  |

***********